|  | Natural Resources Canada | Ressources naturelles Canada |  |
| --- | --- | --- | --- |

CanVec

Product Distribution Formats

Edition 1.0

2007-03-01

Natural Resources Canada

Earth and Sciences Sector

Centre for topographic information

Customer Support Group

2144, King Street West, Suite 010

Sherbrooke, Quebec, Canada

J1J 2E8

Telephone: +01-819-564-4857

1-800-661-2638 (Canada and USA)

Fax: +01-819-564-5698

E-mail: NTDB@NRCan.gc.ca

URL: www.GeoGratis.gc.ca

Copyright notice

© Her Majesty the Queen in Right of Canada, Department of Natural Resources.

All rights reserved.

RELEASES HISTORY

| Date | Version | Description |
| --- | --- | --- |
| 03/2007 | 1.0 | Initial version |

TABLE OF CONTENTS

1	OVERVIEW	1

2	PRODUCT IDENTIFICATION	1

3	DISTRIBUTION FORMATS IDENTIFICATION	1

3.1	GML (Geography Markup Language)	1

3.2	Shape	1

4	DISTRIBUTION FILES IDENTIFICATION	2

4.1	GML file names	2

4.2	Shape file names	2

4.3	Metadata files	3

4.4	List of distribution files names (by alphabetical order)	4

5	ATTRIBUTES IDENTIFICATION	7

5.1	Attributes common to all entities (except those of GeoBase)	7

5.2	Attributes common to all GeoBase entities	7

5.3	Attributes specific to entities	8

APPENDIX A: List of distribution file names (sorted by theme)	12

# OVERVIEW

Geospatial data of CanVec product are available in two distribution formats: GML and Shape.  This document provides only the name of the entities (or events), the name and the data type of the attributes and the file names for each distribution format.  The CanVec Feature Catalogue describes in details the entities (or events), attributes, associations and relations between entities (or events).

# PRODUCT IDENTIFICATION

Name:	CanVec

Version:	1.0

Date:	2007-03-01

Standard:	CanVec: Data product specifications - Edition 1.0 (2007-03-01)

Feature catalogue:	CanVec: Feature catalogue - Edition 1.0 (2007-03-01)

# DISTRIBUTION FORMATS IDENTIFICATION

## GML (Geography Markup Language)

Name:	GML

Version:	02

Date:	20 February 2001

Specifications:	Geography Markup Language (GML), version 2.0, OGC Recommendation Paper, 20 February 2001, OGC Document Number 01-029 (http://portal.opengeospatial.org/files/?artifact_id=1034)

## Shape

Name:	Shape

Version:	01

Date:	July 1998

Specifications:	ESRI Shapefile, a Technical Description, on ESRI white paper (July 1998) (http://www.esri.com/library/whitepapers/pdfs/shapefile.pdf)

# DISTRIBUTION FILES IDENTIFICATION

## GML file names

CanVec entities distributed in GML format are grouped into dataset files.  The name of the GML file is described as follow:

<identifier>_<edition>_<version>.gml

- <identifier> =	National Topographic System (NTS) identifier.  The identifier format is 999a99.

- <edition> =	Dataset edition number.

- <version> = 	Dataset version number.

- .gml =	Extension of file name.

Example:

- 031g01_01_00.gml	(Entities of dataset 031g01, edition 1, version 0).

A XML schema (XSD file) is also delivered with GML data.  This file defines, in a structure manner, the type of content, the syntax and the semantic of XML (and GML) documents.  The name of this file is <identifier>_<edition>_<version>.xsd and is referenced by each GML file.

## Shape file names

The entities of the product distributed in Shape format are divided according to their geometric representation. The name of the Shape files is described as follow:

<identifier>_<edition>_<version>_<THEME CODE>_<generic code>_<geometry code>.shp

- <identifier> =	National Topographic System (NTS) identifier.  The identifier format is 999a99.

- <edition> =	Dataset edition number.

- <version> = 	Dataset version number.

- <THEME CODE> =	Theme code written in uppercase.  Possible codes are: BS, EN, FO, HD, IC, LI, LX, SS, TO, TR, VE.

- <generic code> =	Generic code of 7 digits corresponding to each entity identified in Feature catalogue.

- <geometry code> =	Code indicating the geometric representation of the entity.  Possible values are: 0 (point), 1 (line), 2 (area).

- .shp =	Extension of main geometry file name.

In Shape format, there are also three other files associated with the main geometry file of an entity:

- an attribute file (.dbf for dBASE® file);

- a spatial index file (.shx) containing offset (relative position) for each records of main geometry file;

- a projection file (.prj) storing information about the reference system used and the parameters of the cartographic projection.

Examples:

- 031g01_1_0_FO_1030009_1.shp	(Entity Contour of theme Landform and is a line geometry for dataset 031g01, edition 1, version 0);

- 116b03_1_0_VE_1240009_2.shp	(Entity Wooded area of theme Vegetation and is an area geometry for dataset 116b03, edition 1, version 0).

## Metadata files

Four metadata files are distributed with each dataset of CanVec product.  Two are delivered in FGDC/XML format (in English and in French) and two others according to FGDC/HTML format.  The name of the metadata is described as follow:

canvec_<identifier>_<edition>_<version>_fgdc_<code language>.<format>

- canvec =	Fixed product acronym name.

- <identifier> =	National Topographic System (NTS) identifier.  The identifier format is 999a99.

- <edition> =	Dataset edition number.

- <version> = 	Dataset version number.

- fgdc =	Metadata file format according to CSDGM standard of the Federal Geographic Data Committee (FGDC).

- <code langue> =	Metadata ISO code language written in lowercase: fr (French), en (English).

- <format> =	Extension of file name (xml or html).

Examples:

- canvec_031g01_1_0_fgdc_en.xml	(English metadata file for dataset 031g01, edition 1, version 0 in FGDC/XML format)

- canvec_031g01_1_0_fgdc_fr.html	(French metadata file for dataset 031g01, edition 1, version 0 in FGDC/HTML format)

## List of distribution files names (by alphabetical order)

The file name in Shape format is always preceded by: <identifier>_<edition>_<version>_.  The extension of the file name depends directly of the distribution format.  Furthermore, the name of each entity in the GML file format corresponds to the name of the Shape file indicated in the following table.

The following list is also sorted by theme (see Appendix A).

| Feature catalogue Entity name | Theme | GML – Entity name Shape – File name | Geometry |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Point | Line | Area |
| Amusement park | LX | LX_2260009 |  |  | _2 |
| Auto wrecker | IC | IC_2360009 |  |  | _2 |
| Blocked passage [GeoBase] | TR | TR_1780009 | _0 |  |  |
| Botanical garden | LX | LX_2200009 |  |  | _2 |
| Building | BS | BS_2010009 | _0 |  | _2 |
| Camp | LX | LX_2030009 | _0 |  |  |
| Campground | LX | LX_2480009 | _0 |  | _2 |
| Cemetery | LX | LX_1000039 | _0 |  | _2 |
| Chimney | BS | BS_2060009 | _0 |  |  |
| Coastal water | HD | HD_1150009 |  |  | _2 |
| Contour | FO | FO_1030009 |  | _1 |  |
| Contour imperial | FO | FO_2570009 |  | _1 |  |
| Cross | BS | BS_2120009 | _0 |  |  |
| Cut line | VE | VE_2290009 |  | _1 |  |
| Designated area | LX | LX_1000059 | _0 | _1 | _2 |
| Domestic waste | IC | IC_1360019 |  |  | _2 |
| Drive-in theatre | LX | LX_2070009 | _0 |  | _2 |
| Elevation point | FO | FO_1200009 | _0 |  |  |
| Elevation point imperial | FO | FO_2610009 | _0 |  |  |
| Esker | FO | FO_1080029 |  | _1 |  |
| Exhibition ground | LX | LX_2510009 |  |  | _2 |
| Extraction area | IC | IC_1350039 | _0 |  | _2 |
| Ferry connection segment [GeoBase] | TR | TR_1750009 |  | _1 |  |
| Footbridge | LX | LX_2280009 |  | _1 |  |
| Fort | LX | LX_1000049 |  |  | _2 |
| Gas and oil facilities | EN | EN_1360049 | _0 |  | _2 |
| Glacial debris undifferentiated | FO | FO_1080039 |  |  | _2 |
| Golf course | LX | LX_1000089 |  |  | _2 |
| Golf drining range | LX | LX_2500009 | _0 |  | _2 |
| Historical site/Point of interest | LX | LX_2220009 | _0 |  |  |
| Hydrographic obstacle entity | HD | HD_1460009 | _0 | _1 | _2 |
| Industrial and commercial area | IC | IC_1360039 | _0 |  | _2 |

| Feature catalogue Entity name | Theme | GML – Entity name Shape – File name | Geometry |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Point | Line | Area |
| Industrial solid waste | IC | IC_1360029 | _0 |  | _2 |
| Island | HD | HD_1490009 |  |  | _2 |
| Junction [GeoBase] | TR | TR_1770009 | _0 |  |  |
| Landform | FO | FO_1080019 |  |  | _2 |
| Lookout | LX | LX_1000019 | _0 |  | _2 |
| Lumber yard | IC | IC_2110009 |  |  | _2 |
| Manmade hydrographic entity | HD | HD_1450009 | _0 | _1 | _2 |
| Marina | LX | LX_1000069 | _0 |  |  |
| Mine | IC | IC_1350049 | _0 |  | _2 |
| Mining area | IC | IC_2600009 | _0 |  |  |
| Moraine | FO | FO_1080049 |  |  | _2 |
| Navigational aid | BS | BS_1250009 | _0 |  |  |
| NTS50K boundary polygon | LI | LI_1210009 |  |  | _2 |
| Palsa bog | SS | SS_1320029 |  |  | _2 |
| Parabolic antenna | BS | BS_2000009 | _0 |  |  |
| Park/sports field | LX | LX_2270009 |  |  | _2 |
| Peat cutting | IC | IC_1350059 |  |  | _2 |
| Permanent snow and ice | HD | HD_1140009 |  |  | _2 |
| Picnic site | LX | LX_2490009 | _0 |  | _2 |
| Pingo | FO | FO_1080079 | _0 |  |  |
| Pipeline | EN | EN_1180009 |  | _1 |  |
| Pipeline (Sewage/liquid waste) | BS | BS_2310009 |  | _1 |  |
| Pit | IC | IC_1350019 |  |  | _2 |
| Power transmission line | EN | EN_1120009 |  | _1 |  |
| Quarry | IC | IC_1350029 |  |  | _2 |
| Railway | TR | TR_1020009 |  | _1 |  |
| Residential area | BS | BS_1370009 |  |  | _2 |
| Road segment [GeoBase] | TR | TR_1760009 |  | _1 |  |
| Ruins | LX | LX_2400009 | _0 |  | _2 |
| Runway | TR | TR_1190009 | _0 |  | _2 |
| Sand | FO | FO_1080059 |  |  | _2 |
| Saturated soil | SS | SS_1320039 |  |  | _2 |
| Shrine | LX | LX_2210009 | _0 |  |  |
| Silo | BS | BS_2440009 | _0 |  |  |
| Single line watercourse | HD | HD_1470009 |  | _1 |  |
| Ski centre | LX | LX_1000029 | _0 |  |  |
| Sports track/Race track | LX | LX_1000079 |  | _1 | _2 |

| Feature catalogue Entity name | Theme | GML – Entity name Shape – File name | Geometry |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Point | Line | Area |
| Stadium | LX | LX_2460009 |  |  | _2 |
| String bog | SS | SS_1320059 |  |  | _2 |
| Tank | BS | BS_2080009 | _0 |  | _2 |
| Toll point [GeoBase] | TR | TR_1790009 | _0 |  |  |
| Toponym | TO | TO_2520009 | _0 |  |  |
| Tower | BS | BS_2530009 | _0 |  |  |
| Trail | LX | LX_2420009 |  | _1 |  |
| Transformer station | EN | EN_1360059 | _0 |  | _2 |
| Transmission line | BS | BS_2230009 |  | _1 |  |
| Tundra polygon | FO | FO_1080069 |  |  | _2 |
| Tundra pond | SS | SS_1320019 |  |  | _2 |
| Turntable | TR | TR_2320009 | _0 |  |  |
| Underground reservoir | BS | BS_2380009 | _0 |  | _2 |
| Valve | EN | EN_1340009 | _0 |  |  |
| Wall/fence | BS | BS_2240009 |  | _1 |  |
| Waterbody | HD | HD_1480009 |  |  | _2 |
| Well | BS | BS_2350009 | _0 |  |  |
| Wetland | SS | SS_1320049 |  |  | _2 |
| Wind-operated device | EN | EN_2170009 | _0 |  |  |
| Wooded area | VE | VE_1240009 |  |  | _2 |
| Zoo | LX | LX_2560009 |  |  | _2 |

# ATTRIBUTES IDENTIFICATION

The attribute common to all or a group of entities for CanVec product are identified in the first two tables. Attributes specific to each entity are presented in following subsections.  The data type in GML format is always TEXT (or STRING of characters).  The data type for Shape is either: C(c) for character or N(n,d) for number (c = number of characters, n = total number of digits, d = number of digits in decimal).

## Attributes common to all entities (except those of GeoBase)

| Feature catalogue Entity name | Feature catalogue Attribute name | GML and Shape Attribute name | Shape Data type |
| --- | --- | --- | --- |
| All entities except: - Blocked passage - Ferry connection segment - Junction - Road segment - Toll point | Dataset name | DATANAME | C(10) |
|  | Feature identifier | ID | C(64) |
|  | Planimetric accuracy | ACCURACY | N(10,0) |
|  | Provider | PROVIDER | N(3,0) |
|  | Specification code | CODE | N(10,0) |
|  | Theme | THEME | C(2) |
|  | Validity date | VALDATE | C(10) |

## Attributes common to all GeoBase entities

| Feature catalogue Entity name | Feature catalogue Attribute name | GML and Shape Attribute name | Shape Data type |
| --- | --- | --- | --- |
| Blocked passage Ferry connection segment Junction Road segment Toll point | Acquisition or revision date* | ACQREVDATE | C(7) |
|  | Acquisition technique* | ACQTECH | N(3,0) |
|  | Dataset name | DATANAME | C(10) |
|  | Feature identifier | ID | C(64) |
|  | Planimetric accuracy* | ACCURACY | N(10,0) |
|  | Provider acquisition* | ACQPROVDR | N(3,0) |
|  | Source identifier | SOURCEID | C(32) |
|  | Specification code | CODE | N(10,0) |
|  | Standards version | STDVER | C(10) |
|  | Theme | THEME | C(2) |

* Attribute not available for entity Junction.

## Attributes specific to entities

| Feature catalogue Entity name | Feature catalogue Attribute name | GML and Shape Attribute name | Shape Data type |
| --- | --- | --- | --- |
| Blocked passage [GeoBase] | Blocked passage type | TYPE | N(3,0) |
|  | Network linear element NID | ROADNID | C(32) |
| Building | Building function | FUNCTION | N(3,0) |
|  | Orientation | ORIENTATIO | N(31,8) |
| Coastal water | Type of coastal phenomenon | TYPE | N(3,0) |
| Contour | Altimetric accuracy | ALTIACCU | N(10,0) |
|  | Contour generation | GENERATION | N(3,0) |
|  | Contour type | TYPE | N(3,0) |
|  | Geometric representation qualifier | GEOMREP | N(3,0) |
|  | Hypsometric value | ELEVATION | N(10,0) |
| Contour imperial | Altimetric accuracy | ALTIACCU | N(10,0) |
|  | Contour generation | GENERATION | N(3,0) |
|  | Contour type | TYPE | N(3,0) |
|  | Geometric representation qualifier | GEOMREP | N(3,0) |
|  | Hypsometric value | ELEVATION | N(10,0) |
| Cut line | Cut line type | TYPE | N(3,0) |
| Drive-in theatre | Orientation | ORIENTATIO | N(31,8) |
| Elevation point | Altimetric accuracy | ALTIACCU | N(10,0) |
|  | Elevation point type | TYPE | N(3,0) |
|  | Hypsometric value | ELEVATION | N(10,0) |
| Elevation point imperial | Altimetric accuracy | ALTIACCU | N(10,0) |
|  | Elevation point type | TYPE | N(3,0) |
|  | Hypsometric value imperial | ELEVATION | N(10,0) |
| Exhibition ground | Exhibition ground type | TYPE | N(3,0) |
| Ferry connection segment [GeoBase] | Ferry connection segment ID | FERRYSEGID | N(10,0) |
|  | National road class | NATRDCLASS | N(3,0) |
|  | Route name English 1 | RTENAME1En | C(100) |
|  | Route name English 2 | RTENAME2En | C(100) |
|  | Route name English 3 | RTENAME3En | C(100) |
|  | Route name English 4 | RTENAME4En | C(100) |
|  | Route name French 1 | RTENAME1Fr | C(100) |
|  | Route name French 2 | RTENAME2Fr | C(100) |
|  | Route name French 3 | RTENAME3Fr | C(100) |
|  | Route name French 4 | RTENAME4Fr | C(100) |
|  | Route number 1 | RTeNUMBER1 | C(10) |
|  | Route number 2 | RTeNUMBER2 | C(10) |

| Feature catalogue Entity name | Feature catalogue Attribute name | GML and Shape Attribute name | Shape Data type |
| --- | --- | --- | --- |
| Ferry connection segment [GeoBase] continued | Route number 3 | RTeNUMBER3 | C(10) |
|  | Route number 4 | RTeNUMBER4 | C(10) |
|  | Route number 5 | RTeNUMBER5 | C(10) |
| Hydrographic obstacle entity | Obstacle type | TYPE | N(3,0) |
| Island | Coastal island | COASTAL | N(3,0) |
|  | Sand island | SAND | N(3,0) |
| Junction [GeoBase] | Exit number | EXITNO | C(10) |
|  | Junction type | TYPE | N(3,0) |
| Manmade hydrographic entity | Manmade status | STATUS | N(3,0) |
|  | Manmade type | TYPE | N(3,0) |
| Mining area | Mining area nature | NATURE | N(3,0) |
|  | Mining area status | STATUS | N(3,0) |
|  | Mining area type | TYPE | N(3,0) |
| Navigational aid | Navigational aid type | TYPE | N(3,0) |
| NTS50K boundary polygon | Characteristic of the tiling polygon | TILING | N(3,0) |
|  | Flooded polygon | FLOODED | N(3,0) |
| Parabolic antenna | Antenna type | TYPE | N(3,0) |
| Permanent snow and ice | Permanent snow and ice type | TYPE | N(3,0) |
| Pipeline | Number of pipelines | NOPIPES | N(3,0) |
|  | Pipeline product | PRODUCT | N(3,0) |
|  | Pipeline relationship to ground | RELGROUND | N(3,0) |
| Pipeline (sewage/liquid waste) | Pipeline product | PRODUCT | N(3,0) |
|  | Pipeline relationship to ground | RELGROUND | N(3,0) |
| Power transmission line | Number of power transmission line | NOLINES | N(3,0) |
|  | Power transmission line type | TYPE | N(3,0) |
| Railway | Competence | COMPETENCE | N(3,0) |
|  | Railway gauge | GAUGE | N(3,0) |
|  | Railway status | STATUS | N(3,0) |
|  | Railway support | SUPPORT | N(3,0) |
|  | Railway type | TYPE | N(3,0) |
| Road segment [GeoBase] | Exit number | EXITNO | C(10) |
|  | National road class | NATRDCLASS | N(3,0) |
|  | Number of lanes | NOLANES | N(3,0) |
|  | Paved road surface type | PAVSURF | N(3,0) |
|  | Pavement status | PAVSTATUs | N(3,0) |
|  | Road segment ID | ROADSEGID | N(10,0) |
|  | Route name English 1 | RTENAME1En | C(100) |

| Feature catalogue Entity name | Feature catalogue Attribute name | GML and Shape Attribute name | Shape Data type |
| --- | --- | --- | --- |
| Road segment [GeoBase] continued | Route name English 2 | RTENAME2En | C(100) |
|  | Route name English 3 | RTENAME3En | C(100) |
|  | Route name English 4 | RTENAME4En | C(100) |
|  | Route name French 1 | RTENAME1Fr | C(100) |
|  | Route name French 2 | RTENAME2Fr | C(100) |
|  | Route name French 3 | RTENAME3Fr | C(100) |
|  | Route name French 4 | RTENAME4Fr | C(100) |
|  | Route number 1 | RTENUMBER1 | C(10) |
|  | Route number 2 | RTENUMBER2 | C(10) |
|  | Route number 3 | RTENUMBER3 | C(10) |
|  | Route number 4 | RTENUMBER4 | C(10) |
|  | Route number 5 | RTENUMBER5 | C(10) |
|  | Structure ID | STRUCID | C(32) |
|  | Structure name English | STRUNaMEEN | C(100) |
|  | Structure name French | STRUNAMEFR | C(100) |
|  | Structure type | STRUCTYPE | N(3,0) |
|  | Unpaved road surface type | UNPAVSURF | N(3,0) |
| Ruins | Orientation | ORIENTATIO | N(31,8) |
| Runway | Runway status | STATUS | N(3,0) |
|  | Runway type | TYPE | N(3,0) |
| Single line watercourse | Isolated | ISOLATED | N(3,0) |
|  | Permanency | PERMANENCY | N(3,0) |
|  | Water definition | DEFINITION | N(3,0) |
| Tank | Orientation | ORIENTATIO | N(31,8) |
|  | Tank type | TYPE | N(3,0) |
|  | Tank use | USE | N(3,0) |
| Toll point [GeoBase] | Network linear element NID | ROADNID | C(32) |
|  | Toll point type | TYPE | N(3,0) |
| Toponym | CGNDB identifier | CGNDBID | C(5) |
|  | Name | TOPONAME | C(100) |
|  | Toponym type | TYPE | N(3,0) |
| Tower | Tower function | FUNCTION | N(3,0) |
| Trail | Trail function | FUNCTION | N(3,0) |
| Transmission line | Number of transmission line | NOLINES | N(3,0) |
|  | Transmission line function | FUNCTION | N(3,0) |
|  | Transmission line location | LOCATION | N(3,0) |
| Wall/Fence | Wall/Fence type | TYPE | N(3,0) |

| Feature catalogue Entity name | Feature catalogue Attribute name | GML and Shape Attribute name | Shape Data type |
| --- | --- | --- | --- |
| Waterbody | Isolated | ISOLATED | N(3,0) |
|  | Permanency | PERMANENCY | N(3,0) |
|  | Waterbody definition | DEFINITION | N(3,0) |
| Well | Well type | TYPE | N(3,0) |
| Wooded area | Wooded area type | TYPE | N(3,0) |

APPENDIX A: List of distribution file names (sorted by theme)

| Feature catalogue Entity name | Theme | GML – Entity name Shape – File name | Geometry |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Point | Line | Area |
| Navigational aid | BS | BS_1250009 | _0 |  |  |
| Residential area | BS | BS_1370009 |  |  | _2 |
| Parabolic antenna | BS | BS_2000009 | _0 |  |  |
| Building | BS | BS_2010009 | _0 |  | _2 |
| Chimney | BS | BS_2060009 | _0 |  |  |
| Tank | BS | BS_2080009 | _0 |  | _2 |
| Cross | BS | BS_2120009 | _0 |  |  |
| Transmission line | BS | BS_2230009 |  | _1 |  |
| Wall/fence | BS | BS_2240009 |  | _1 |  |
| Pipeline (Sewage/liquid waste) | BS | BS_2310009 |  | _1 |  |
| Well | BS | BS_2350009 | _0 |  |  |
| Underground reservoir | BS | BS_2380009 | _0 |  | _2 |
| Silo | BS | BS_2440009 | _0 |  |  |
| Tower | BS | BS_2530009 | _0 |  |  |
| Power transmission line | EN | EN_1120009 |  | _1 |  |
| Pipeline | EN | EN_1180009 |  | _1 |  |
| Valve | EN | EN_1340009 | _0 |  |  |
| Gas and oil facilities | EN | EN_1360049 | _0 |  | _2 |
| Transformer station | EN | EN_1360059 | _0 |  | _2 |
| Wind-operated device | EN | EN_2170009 | _0 |  |  |
| Contour | FO | FO_1030009 |  | _1 |  |
| Landform | FO | FO_1080019 |  |  | _2 |
| Esker | FO | FO_1080029 |  | _1 |  |
| Glacial debris undifferentiated | FO | FO_1080039 |  |  | _2 |
| Moraine | FO | FO_1080049 |  |  | _2 |
| Sand | FO | FO_1080059 |  |  | _2 |
| Tundra polygon | FO | FO_1080069 |  |  | _2 |
| Pingo | FO | FO_1080079 | _0 |  |  |
| Elevation point | FO | FO_1200009 | _0 |  |  |
| Contour imperial | FO | FO_2570009 |  | _1 |  |
| Elevation point imperial | FO | FO_2610009 | _0 |  |  |
| Permanent snow and ice | HD | HD_1140009 |  |  | _2 |
| Coastal water | HD | HD_1150009 |  |  | _2 |
| Manmade hydrographic entity | HD | HD_1450009 | _0 | _1 | _2 |
| Hydrographic obstacle entity | HD | HD_1460009 | _0 | _1 | _2 |

| Feature catalogue Entity name | Theme | GML – Entity name Shape – File name | Geometry |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Point | Line | Area |
| Single line watercourse | HD | HD_1470009 |  | _1 |  |
| Waterbody | HD | HD_1480009 |  |  | _2 |
| Island | HD | HD_1490009 |  |  | _2 |
| Pit | IC | IC_1350019 |  |  | _2 |
| Quarry | IC | IC_1350029 |  |  | _2 |
| Extraction area | IC | IC_1350039 | _0 |  | _2 |
| Mine | IC | IC_1350049 | _0 |  | _2 |
| Peat cutting | IC | IC_1350059 |  |  | _2 |
| Domestic waste | IC | IC_1360019 |  |  | _2 |
| Industrial solid waste | IC | IC_1360029 | _0 |  | _2 |
| Industrial and commercial area | IC | IC_1360039 | _0 |  | _2 |
| Lumber yard | IC | IC_2110009 |  |  | _2 |
| Auto wrecker | IC | IC_2360009 |  |  | _2 |
| Mining area | IC | IC_2600009 | _0 |  |  |
| NTS50K boundary polygon | LI | LI_1210009 |  |  | _2 |
| Lookout | LX | LX_1000019 | _0 |  | _2 |
| Ski centre | LX | LX_1000029 | _0 |  |  |
| Cemetery | LX | LX_1000039 | _0 |  | _2 |
| Fort | LX | LX_1000049 |  |  | _2 |
| Designated area | LX | LX_1000059 | _0 | _1 | _2 |
| Marina | LX | LX_1000069 | _0 |  |  |
| Sports track/Race track | LX | LX_1000079 |  | _1 | _2 |
| Golf course | LX | LX_1000089 |  |  | _2 |
| Camp | LX | LX_2030009 | _0 |  |  |
| Drive-in theatre | LX | LX_2070009 | _0 |  | _2 |
| Botanical garden | LX | LX_2200009 |  |  | _2 |
| Shrine | LX | LX_2210009 | _0 |  |  |
| Historical site/Point of interest | LX | LX_2220009 | _0 |  |  |
| Amusement park | LX | LX_2260009 |  |  | _2 |
| Park/sports field | LX | LX_2270009 |  |  | _2 |
| Footbridge | LX | LX_2280009 |  | _1 |  |
| Ruins | LX | LX_2400009 | _0 |  | _2 |
| Trail | LX | LX_2420009 |  | _1 |  |
| Stadium | LX | LX_2460009 |  |  | _2 |
| Campground | LX | LX_2480009 | _0 |  | _2 |
| Picnic site | LX | LX_2490009 | _0 |  | _2 |
| Golf drining range | LX | LX_2500009 | _0 |  | _2 |

| Feature catalogue Entity name | Theme | GML – Entity name Shape – File name | Geometry |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Point | Line | Area |
| Exhibition ground | LX | LX_2510009 |  |  | _2 |
| Zoo | LX | LX_2560009 |  |  | _2 |
| Tundra pond | SS | SS_1320019 |  |  | _2 |
| Palsa bog | SS | SS_1320029 |  |  | _2 |
| Saturated soil | SS | SS_1320039 |  |  | _2 |
| Wetland | SS | SS_1320049 |  |  | _2 |
| String bog | SS | SS_1320059 |  |  | _2 |
| Toponym | TO | TO_2520009 | _0 |  |  |
| Railway | TR | TR_1020009 |  | _1 |  |
| Runway | TR | TR_1190009 | _0 |  | _2 |
| Ferry connection segment [GeoBase] | TR | TR_1750009 |  | _1 |  |
| Road segment [GeoBase] | TR | TR_1760009 |  | _1 |  |
| Junction [GeoBase] | TR | TR_1770009 | _0 |  |  |
| Blocked passage [GeoBase] | TR | TR_1780009 | _0 |  |  |
| Toll point [GeoBase] | TR | TR_1790009 | _0 |  |  |
| Turntable | TR | TR_2320009 | _0 |  |  |
| Wooded area | VE | VE_1240009 |  |  | _2 |
| Cut line | VE | VE_2290009 |  | _1 |  |