- Observer's Daily Comments -

CLIENT: NORANDA INC.                         AREA: HALFMILE LAKE 3D                               DATE: 18/ SEPT/ 98

CONTRACTOR: WESTERN GEOPHYSICAL          PARTY NO: 779         JOB NO: 98-005       OBSERVERS: G. MITSCHKE / R. DAVIS

----------------------------------------------------------------------------------------------------------------------

WEATHER: CLEAR, LIGHT WINDS, 4 C. TO 19 C.

CAP SIM TEST; PELTON S/N-020, FILE 404.

CAP SIM TEST; PELTON S/N-038, FILE 405.

CAP SIM TEST; PELTON S/N-026, FILE 406.

RECEIVER LINES PICKED UP TODAY:

R-1 - STATIONS 334 TO 305.				R-9 - STATIONS 334 TO 302.

R-3 - STATIONS 334 TO 302.				R-11 - STATIONS 334 TO 302.

R-5 - STATIONS 334 TO 302.				R-13 - STATIONS 334 TO 302.

R-7 - STATIONS 334 TO 302.				R-15 - STATIONS 334 TO 302.

RECEIVER LINES LAYED OUT TODAY:

R-11 - STATIONS 119 TO 101. (COMPLETE)

R-13 - STATIONS 119 TO 101. (COMPLETE)

HELICOPTER PICKED UP (15) HELIBAGS OFF THE BACK OF THE SPREAD AND BROUGHT THEM TO STAGING TO BE BROKE DOWN

AND CHECKED FOR ANY BAD EQUIPMENT.

SOURCE LINE SHOT ON TODAY:

S-14 - S.P.'s 101 TO 147.				S-6 - S.P.'s 101 TO 147.

S-12 - S.P.'s 101 TO 147.				S-4 - S.P.'s 104 TO 147.

S-10 - S.P.'s 101 TO 147.				S-2 - S.P.'s 103 TO 147.

S-8 - S.P.'s 101 TO 147.

*** OMIT FILES 412, 539, 579, 583, 680, 703 AND 708; MISFIRES.

*** WIND INTENSITY INCREASED IN THE AFTERNOON. WIND GUSTS EFFECTING RECORDS. STOPPED RECORDING UNTIL IT WAS CALM.

*** NOTE: S-2; ENERGY LEVELS WERE CONSIDERABLY STRONGER. THE SHOOTER CONFIRMED THE DIFFERENCE

IN ENERGY WHEN HE STARTED SHOOTING S-4.

PROFILES FOR THE DAY: 317        				   		  PROFILES FOR THE PROSPECT: 702

TOTAL TRAVEL TIME: 2.0 HRS       FIELD TIME: 12.0  HRS         DOWN TIME: 0.0  HRS        TOTAL CHARGEABLE HRS: 14.0