Borehole seismic imaging of near-vertical structures: A case history

David Eaton*, Matt Salisbury, David Forsyth and Wouter Bleeker, Geological Survey of Canada;

Bernd Milkereit, Kiel University; Sean Guest, Koninklijke Shell Explaortie en Produktie Laboratorium;

Doug Schmitt, University of Alberta; Dean Crick, Falconbridge Ltd.

Summary

Conventional seismic methods are generally not effective for imaging geological structures with very steep dips, a common situation in some mining camps. Deep boreholes provide an alternative acquisition datum for seismic imaging in this type of setting. At the giant Kidd Creek Cu-Zn deposit, the Archean volcanic stratigraphy has been folded and overturned so that most units are nearly vertical. Borehole logging and laboratory measurements conducted here show that certain stratigraphic contacts, especially those where massive-sulphide deposits and felsic host rocks are juxtaposed, are characterized by large impedance contrasts and are thus good candidates as seismic reflectors. To investigate this possiblity, a series of borehole-seismic profiling experiments were conducted in an area of known sulphide mineralization. Several source configurations were used, yielding both vertical and horizontal seismic images of the volcanic stratigraphy. In addition to observing reflections that correlate with stratigraphic contacts, prominent seismic anomalies were detected and have been correlated with a known massive-sulphide deposit.

Introduction

Recent studies suggest that seismic methods have excellent potential as a geophysical tool for deep mineral exploration in crystalline terranes (Salisbury et al., 1996; Milkereit et al., 1996; Eaton et al., 1996). Geological settings characterized by steep dips (> 65o), however, are often problematic for conventional surface seismic methods, because a large fraction of the reflected energy propagates (initially) either horizontally or downwards. In cases where seismic velocities increase rapidly with depth, turning-ray methods (Hale et al., 1992) offer a possible solution; in practice, however, application of these techniques is effectively limited to young sedimentary basins where suitable velocity gradients exist. A different approach is therefore needed to resolve steeply dipping structures in a hard-rock environment.

Vertical seismic profiling (VSP) provides an alternative solution, since subsurface receivers are more optimally situated than surface arrays for intercepting reflected rays that are nearly horizontal. An additional benefit of borehole recordings is that they often contain higher frequencies than surface data, because the reflection raypaths have only one leg through highly attenuating near-surface layers, instead of two. Nonhydrocarbon VSP experiments are relatively uncommon. Several studies related to continental scientific drilling programs, such as the Cajon Pass drill hole in the southwestern United States (Rector, 1988) and the KTB drill hole in southern Germany (Luschen et al., 1991) have been reported in the literature. In a more recent case, Miao et al. (1995) analyzed VSP data near Sudbury, Canada, and used their results to establish a stratigraphic tie with a coincident surface seismic profile.

This paper presents recently acquired three-component borehole seismic data from the vicinity of the giant Kidd Creek Cu-Zn deposit in the Abitibi greenstone belt, Canada (Fig. 1). The data were collected to assess the potential of borehole seismic methods for mineral exploration in an area of very steep dips. The initial survey was conducted using a single source location and a large number of receiver positions, and yielded a seismic image in the format of a vertical cross section. The second survey, conducted several months later, made use of a large number of source locations and a compact receiver array. This data was processed to yield images representing approximately horizontal sections of the target area.

Geological Setting

The study area is situated in the Canadian Shield within the Abitibi greenstone belt (Fig. 1). In this area, the Archean volcanic stratigraphy consists of packages of basaltic, ultramafic and felsic units of variable thickness (Walker et al., 1975). The Kidd Creek orebody is a volcanogenic massive sulphide (VMS) deposit that occurs within a felsic marker horizon, informally known as the Mine Rhyolite. All rocks in the area have been affected by regional greenschist metamorphism, and most are situated in fold limbs such that their current attitude at the surface, and in at least the uppermost few kilometers of the crust, is approximately vertical.

Several other rhyolites occur locally that have a recognized potential to host economic ore deposits. The borehole seismic surveys are focused on one of these units, the North Rhyolite, a relatively thin (30 m) felsic unit which extends in an arc northwest of the mine (Fig. 1). This unit dips steeply towards the northeast to a vertical depth of at least 1.5 km. It is flanked to the southwest by serpentinized ultramafic rocks and gabbro and to the northeast by basaltic flows.

Massive pyrite-rich sulphide lenses occur locally at the stratigraphic top of the North Rhyolite, or are intercalated within the felsic volcanic package. Detailed laboratory velocity and density measurements (Salisbury et al., 1996) indicate that the contact between these massive sulphides and host Rhyolite units, in addition to several other stratigraphic contacts in the Kidd Creek camp, are characterized by large reflection coefficients. These findings are confirmed by logging results from a borehole that intersects many of the dipping panels of volcanic rocks in the camp, in addition to a 10-m interval of massive sulphide (borehole "B", Fig. 1). A simple synthetic seismogram, computed by convolving a 40-Hz Ricker wavelet with the reflectivity series determined from the velocity and density logs from borehole "B"  shows a number of good candidate seismic reflectors that include: 1) the top of the North Rhyolite (R), 2) the massive-sulphide zone (S), and 3) the top of a gabbro sill unit (G). Taken together, the large impedance contrasts,  abrupt nature of the geologic contacts and the lateral continuity of the volcanic stratigraphy all imply that seismic methods may be a viable exploration technique in the Kidd Creek camp.

Single-source VSP

To test if the candidate seismic reflectors identified from borehole logging could be observed seismically, a series of borehole seismic experiments were conducted in February 1994. These experiments included a single-source VSP recorded in borehole "A", as well as several other borehole seismic surveys that are not considered in this paper. The 3-component data were acquired using a Geosource tool and a Bison 12-channel recorder. Note that the NQ-diameter (6.03 cm) diamond drill hole required specialized equipment, since most deep borehole acqusition systems are designed for much larger diameter holes. The seismic source (large dot in Fig. 1) consisted of small pentolite boosters detonated in a water-filled pit, and was located about 400 m northeast of the borehole collar. Receivers were positioned every 15 m in the borehole, down to the approximate depth level of the North Rhyolite contact at 1030 m.

The main data processing steps applied to the VSP data followed the standard practice for processing offset vertical seismic profile data (e.g. Hardage, 1985). Processing steps included: 1) a notch filter to remove 60 Hz and associated harmonics from the data; 2) rotation of the horizontal components into radial and transverse orientations, such that the energy in a time window centered on the P-wave first break is maximized for the radial component and minimized for the transverse component; 3) removal of downgoing direct arrivals by successive flattening, application of a 13-trace median filter, subtraction, and unflattening; 4) enhancement of the reflected arrivals by local semblance filtering; 5) image reconstruction using a ray-tracing based VSPCDP transform technique (Wyatt and Wyatt, 1984), using a velocity model derived from P-wave direct arrival time picks. This step yields an image that approximates a vertical section in the plane containing both the borehole receivers and the source. This mapping procedure assumes that all reflection raypaths are confined to the image plane.

Fig. 2b shows the processed radial-component VSP data. Reflections are apparent with dips ranging from 70o to 85o, slightly less than the true geologic dip. This mismatch is attributed to lateral velocity variations or anisotropic effects not accounted for in the model. The reflection labelled "R" intersects borehole "A" at the North Rhyolite contact, providing a basis for lithologic correlation of other deeper reflections. Additional control is provided by borehole "B". The synthetic seismic trace for this borehole is shown in Fig. 2b, projected into the VSP image plane. The high-amplitude negative reflection labelled "S" appears to correspond with the massive sulphide deposit intersected by borehole "B". Reflection S builds in amplitude between 200 m and 300 m depth, and contains the largest amplitudes observed in the seismic image. The termination of this reflection above borehole "A" is consistent with the absence of massive sulphides at this stratigraphic level in borehole "A".

Multiple-source VSP

As an additional test of borehole seismic imaging in a setting of steeply dipping structures, a second borehole seismic experiment was conducted in June, 1994. This survey consisted of a line of 83 sources oriented approximately parallel to the strike of the North Rhyolite contact (Fig. 1). For a given source location, successive shots were fired by re-loading 5-m deep overburden holes cased with PVC tubing. 3-component recordings were obtained for a synthetic array of 13 receiver levels, between wireline depths of 477 m and 642 m. The choice of 13 receiver levels was based on experience gained from processing the initial VSP survey, since this number was determined to be optimal for use in median filtering  to remove the downgoing arrivals. This shooting configuration was designed to yield seismic images that approximate horizontal sections through the steeply dipping stratigraphy.

Processing of this dataset followed the sequence established in the earlier survey. Most of the data processing was implemented in the shot-gather domain, with the exception of the final transformation of the data into world coordinates. Modeling tests indicated that, for a given receiver level, the reflection points at various lithologic contacts tend to occur at approximately the same depth but diverge with distance away from the borehole. Moreover, the curvilinear geometry of the North Rhyolite and associated reflection zones is fortuitously oriented such that lines joining each shot-receiver pair intersect the stratigraphy approximately perpendicular to the strike of the contact. Consequently, a simple but very useful image section was produced by mapping each data value to its approximate reflection point as predicted by 2-D ray tracing in the plane containing both the shot and receiver. The image produced by this pseudo 3-D transformation occupies a pie-shaped region, with its tip located at the receiver array (Fig. 1).

Fig. 3 shows the image section produced by applying this transformation to a common-receiver gather of the the transverse component data, for the receiver at a depth of 612 m. The (approximately) horizontal image section extends over a maximum along-strike distance of 650 m, penetrates 700 m beyond the North Rhyolite contact and is at an average depth of 550 m below surface. Note that this depth corresponds to the level at which borehole "B" intersects the massive sulphide lens. The thin blank zones within the seismic image correspond with gaps in the shooting line (Fig. 1).

By correlating arrivals between the multiple-source survey and the previous single-source VSP survey, the reflection from the North Rhyolite can be identified relatively unambiguously . Referring to Fig. 3, the North Rhyolite reflection appears to delineate the boundary between a relatively unreflective region to the northeast and a more reflective region to the southwest. This knowledge of the location of the North Rholite reflection (Fig. 3) helps to constrain the interpretation of other arrivals. For example, the event marked as "Gabbro Sills" in Fig. 3 is interpreted as the interfingering contact between high-impedance gabbro sills and highly altered low-impedance ultramafic rocks.

More significantly, a high-amplitude arrival immediately following the North Rhyolite reflection was evident prior to application of the mapping transformation, with an apparent dip that was discordant with respect to adjacent reflections. Confidence in this feature is enhanced by its observation on 5 adjacent common-receiver gathers. Assuming that the pseudo 3-D mapping algorithm is valid, this seismic anomaly appears to correspond with the location of the massive sulphide lens that was intersected in borehole "B". The key attributes of the anomaly that could be used to recognize similar anomalies in other datasets are: 1) localization in space (i.e. lack of continuity throughout the entire data volume); 2) dip discordancy with respect to adjacent reflections that correlate with stratigraphic units; 3) large amplitude relative to other reflections in the dataset.

Conclusions

Laboratory and in situ measurements indicate that the Kidd Creek volcanic complex is a favourable setting for the application of seismic techniques for deep mineral exploration. In a series of experiments, we have developed new vertical seismic profiling (VSP) techniques for imaging steeply dipping structures. Our initial survey made use of a single source location and receivers every 15 m over the entire length of the borehole. This configuration produced seismic images that approximate vertical sections through the steeply rock units. A subsequent survey was conducted using 83 shot locations in a line parallel to the North Rhyolite, the target unit known to host massive sulphides. Application of a pseudo 3-D transformation to this dataset produced nearly horizontal image sections. For both types of surveys, a seismic anomaly was observed that can be correlated with a known occurrence of massive sulphides. We are continuing to investigate the application of borehole seismic technques for deep mineral exploration in this type of geological setting.

Acknowledgments

This project was jointly funded by Falconbridge Inc. (Kidd Creek Division) and the Geological Survey of Canada through the GSC's Industrial Partners Program. We are grateful to Falconbridge for permission to publish these results. Log data for borehole "B" were were provided by K. Pflug.

References

Eaton, D., Guest, S., Milkereit, B., Bleeker, W., Crick, D.,

Schmitt, D., Salisbury, M., 1996. Seismic imaging of

massive sulphide deposits, part 3: borehole 	seismic

imaging of near-vertical structures, Ec. Geology, in

review.

Hale, D., Hill. N.R., and Stefani, J., 1992. Imaging salt with

turning seismic waves: Geophysics, 57, 1453-1462.

Hardage, R.A., 1985. Vertical seismic profiling part A:

Principles: Geophysical Press, London,  509 pp.

Luschen, E., Sollner, W., and Hohrath, A., 1991. Integrated

P- and S-wave borehole  experiments at the KTB deep

drilling site in the Oberpfalz area: American Geophysical

Union,  Geodynamics Series, 22, 121-133.

Miao, X.G., Moon, W.M., and Milkereit, B., 1995. A

multioffset, three-component VSP study  in  the Sudbury

Basin: Geophysics, 60, 341-353.

Milkereit, B., Eaton, D., Wu, J., Perron, G., Salisbury, M.,

Berrer, E.K., Morrison, G., 1996. Seismic imaging of

massive sulphide deposits, part 2: reflection seismic

profiling, Ec. Geology, in review.

Rector III, James W., 1988. Acquisition and preliminary

analysis of oriented multicomponent multioffset VSP

data: Cajon Pass deep scientific drillhole: Geophys. Res.

Lett., 15, 1061- 1064.

Salisbury, M.H., Milkereit, B. and Bleeker, W., 1996.

Seismic imaging of massive sulphide deposits, part 1:

rock properties: Ec. Geology, in review.

Walker, R.R., Matulich, A., Amos, A.C., Watkins, J.J. and

Maunard, G.W., 1975. The geology of the Kidd Creek

mine. Economic Geology, 70, 80-89.

Wyatt, K.B., and Wyatt, S.B., 1984. Determining subsurface

structure using the vertical seismic profile. In Toksöz,

M.N. and Stewart, R.R., Eds., Vertical seismic profiling,

Part B:  Advanced Concepts. Geophysical Press, London,

p. 148-176.