|  | Sedimentary Rocks | Crystalline Rocks |
| --- | --- | --- |
| Velocity | Vp =f(z) Vp ≺1.8 - 5.5 km/s | Vp ∼ constant Vp ∼ 6.0 km/s |
| Wave Propagation | layered model (geometric optics) | perturbation model (Born Approximation) |
| Velocity-Density relationship | Vp ∼a.ρ 4 (Gardner, 1974) | Vp ∼a.ρ (Birch, 1961) |
| Reflection coefficients | RC ~ f(Vp) | RC~f(ρ) (Salisbury et al., 1995) |
| Signal/noise considerations | Signal: 0 - 120 Hz | Signal 30 - 200 Hz Noise 0 - .30 Hz |