- Table 2.  Seismic acquisition parameters

|  | Line 1 | Lines 2 & 3 |
| --- | --- | --- |
| Line length | 20 km | Line 2 - 22.5 km, line 3 - 12.5 km |
| Number of Channels | 480 | 360 |
| Record Length | 3 s | 3 s |
| Sample Rate | 1 ms | 2 ms |
| Filters | 160 Hz high cut | 164 Hz high cut |
| Shot Interval | 40 m | 80 m |
| Receiver Interval | 10 m | 20 m |
| Nominal fold | 60 | 30 |
| Geophones/group | 9 over 9 m | 6 over 2m |
| Geophone type | Mark Products L10 Spike (14 Hz) | Mark products UM2-A Spike (10 Hz) |
| Spread type | Symmetrical split spread | Symmetrical split spread |
| Output format | SEGY | SEGY |
| Source type | Dynamite, 0.34 kg | Dynamite, 0.34 kg & cone charges |