- Table 3.  Processing Sequence - Manitouwadge Line 1

1) Geometry application, first break picking and trace editing.

2) Refraction Statics 	- Fathom, Green Mountain Geophysics

- 400 m datum, 5650 m/s replacement vel

3) Spectral Balancing	- 35 to 165 Hz, 25 Hz window

4) Front Mute		- mute 35 ms after first break pick

5) CDP gather		- nominal fold 60, maximum fold 115

6) NMO correction	- constant velocity, 55% stretch mute

- Brute Stack

7) Velocity Analysis	- Constant velocity stacks

- Conventional Stack

8) Residual Statics	- Surface consistent, .2 to 1.3 sec window

9) DMO			- Dip Moveout correction using log stretch approach

10) Velocity Analysis	- Constant velocity stacks

- DMO Stack

11) F-X Decon		- Wiener Levinson  (ProMAX)

12) Migration		- Kirkhoff depth migration (ProMAX), interval velocities derived from 			velocity function defined above.

- Migrated Stack

- Table 4.   Processing Sequence - Manitouwadge Lines 2 and 3

1) Geometry application, first break picking and trace editing.

2) Refraction Statics 	- GLI3D, Hampson-Russell

- 340 m datum, 1 layer model, 5650 m/s replacement vel

3) 2-D spatial filtering	- 7 sample median filter at 3250 m/s dip (shear wave removal)

- 4) Air blast attenuation

5) Deconvolution	- minimum phase spiking decon, 100 ms operator length

6) Bandpass filter	- minimum phase Ormsby bandpass, 10-30-130-150 (Hz)

7) Front Mute		- mute 35 ms after first break pick

8) CDP gather		- nominal fold 30

9) NMO correction	- constant velocity, 70% stretch mute

- Brute Stack

10) Velocity Analysis	- Constant velocity stacks and semblance analysis

- Conventional Stack

11) Residual Statics	- maximum power autostatics

12) DMO		- Ensemble DMO in T-X domain

13) Velocity Analysis	- Constant velocity stacks and semblance analysis

- DMO Stack

14) Bamdpass filter	- minimum phase Ormsby bandpass, 10-30-110-120 (Hz)

15) F-X Decon		- Wiener Levinson, 15 traces, 500 ms window

16) Migration		- Kirkhoff depth migration, max dip 60o ,interval velocities derived from 			velocity function defined above.

- Migrated Stack