Audiomagnetotelluric Study of the Nipigon Embayment: Preliminary Results

J.A. Craven1, Shareef, S.1, Hart, T.R. 2, Martí, A.3, and Farquharson, C.4

1Geological Survey of Canada, 615 Booth St., Ottawa, ON

2Ontario Geological Survey, 333 Ramsey Lake Rd, Sudbury, ON

3 University of Barcelona, Barcelona, Spain.

4 Inco Innovation Center, Memorial University, St. John’s, NF

# Abstract

To date, the shallow portion of the Nipigon Embayment has been difficult to image geophysically due to the large overprint of the Archean basement in the gravity and aeromagnetic signatures.  To image the Proterozoic sedimentary and igneous lithologies a large regional audiomagnetotelluric (AMT) survey was undertaken.  Borehole logging and rock petrophysical data are integrated with the AMT results to interpret the models.   A westward thinning sedimentary succession that appears to be offset by en echelon faults, the most predominant of which is the Black Sturgeon River fault, has been directly imaged geophysically for the first time. The AMT imaging also reveals subsurface Nipigon sills seen only in sparse boreholes.  The sills appear to rise in the sedimentary succession. AMT observations of shallowing sills are consistent with seismic observations of sills emplaced in other shallow basins and finite element models of shallow sill emplacement.  The electrical rock properties in the region and relatively flat-lying geometries ensure that AMT is a viable technique for exploration of the Sibley Group and Nipigon sills in the Embayment.

Introduction

The Nipigon Embayment (Figure 1), an area of Mesoproterozoic igneous and sedimentary rocks centred on Lake Nipigon, is believed to be a manifestation of failed rifting associated with the Keweenawan Midcontinent Rift system (e.g. Franklin et al. 1980).  A series of north- and northwest-trending faults displace the underlying Archean rocks of the Wabigoon and Quetico subprovinces, and have been interpreted to be en echelon resulting in the formation of an asymmetric basin or graben (Coates 1972) defining the southwest portion of the Embayment. The relatively flat-lying Mesoproterozoic Sibley Group sedimentary rocks unconformable overly the Archean rocks and are thickest within the southern portion of the half-graben. Four mafic to ultramafic intrusions, and a series of associated thinner sills, intrude the Sibley Group and the Archean rocks. The Nipigon sills are a series of shallow-dipping diabase sills that intrude all other rock types in the Embayment. Recent diamond drilling in one mafic to ultramafic intrusion, the Seagull Intrusion, has identification a number of widely distributed zones of PGE mineralization (Platinum Group Metals Limited 2005).  Two aims of the Lake Nipigon Region Geoscience Initiative (LNRGI) were to determine the potential for additional mafic to ultramafic intrusions and to understand the controls on emplacement of these intrusions.

Figure 1.  Survey location and geology map. Electric strike directions at every fifth site for the given frequency are plotted. For each survey line, a histogram of the strike direction is shown. The red box illustrates the plan view of the model shown in Figure 6.

Although mapped geologically by a number of workers over the past 135 years (e.g. Bell 1870; Hart, 2004), and recently probed geophysical utilizing a number of techniques such as gravity and aeromagnetic (e.g. Ontario Geological Survey, 2004; 2004b), three dimensional models of the sedimentary rocks of the Sibley Group based on this data has had limited success.  A number of cross-sections using the available diamond drill hole information on both a local (e.g. Hart and Tolson 2004) and basin-wide (e.g. Rogala et al. 2005) scales have provided additional insights into the third dimension, but are limited due to the sparse distribution of drill holes in the Embayment. The flat-lying sedimentary and igneous material has proven difficult to image geophysical due to the strong overprint of the Nipigon diabase sills and the underlying Archean rocks.

Sedimentary rocks are porous.  When saturated with groundwater these rocks can be quite conductive and will exhibit a significant electrical contrast with the resistive Archean crystalline basement rocks and intruding Nipigon sills.  Electromagnetic geophysical techniques such as magnetotellurics (MT) are well suited to imaging sedimentary basins provided they have sufficient transmitter power to penetrate to significant depths (Vozoff, 1972).  MT utilizes powerful oscillations in the Earth’s natural magnetic field as the transmitter.   The crust and ionosphere acts as a waveguide and transmits the EM waves initiated by distant thunderstorms and magnetospheric affects about the earth.  The transmitted waves have primarily a vertical component but interactions with the crust set-up horizontal components that can be used to infer the electrical properties of the waveguide.  The ratios of the amplitudes of the electric and magnetic field and their phase relationships can be used to determine the resistivity of the crust.  Recordings of source and scattered EM fields are made using a small crew and the data are processed to a preliminary image of the electrical structure quickly.

This study integrates borehole logging, petrophysical, and magnetotelluric data to image the Sibley Group, mafic and ultramafic intrusions with the overarching goal of providing geometrical constraints on models for the geological evolution of the Nipigon Embayment.  This report and attached PowerPoint presentation file provide a preliminary look at the data and subsurface models.   The primary advantage of the PowerPoint file is to visualize the movie representation of the preliminary 3-D model in the vicinity of the Disraeli intrusion.

Regional Geology

The Nipigon Embayment is an approximately 19 000 km2 area of Mesoproterozoic igneous and sedimentary rocks centred on Lake Nipigon.  Archean rocks of the Wabigoon and Quetico subprovinces, and minor Mesoproterozoic intrusive and volcanic rocks, underlie the later Mesoproterozoic rocks.  The Embayment is bordered to the south by the volcanic rocks of the Osler Group that were deposited along the north edge of the Keweenawan Midcontinent Rift (Lightfoot et al. 1991).  A series of north- and northwest-trending faults localized deposition of the sedimentary rocks and emplacement of the igneous intrusive rocks within the Embayment.

This study examined the southwest portion of the Nipigon Embayment, which is underlain by Archean rocks of the southern Wabigoon and northern Quetico subprovinces.  Most of the Wabigoon Subprovince is composed of a series of 2.95 to 2.70 Ga greenstone belts and 3.0 to 2.69 Ga intrusive rocks (e.g. Tomlinson et al. 2003), with the southern margin consisting of interbanded metavolcanic and metasedimentary rocks (e.g. Devaney and Williams 1989). The greenstone belts are composed of mafic to felsic metavolcanic rocks and interflow clastic and chemical sedimentary rocks, with the eastern portions of the Heaven Lake and Lac des Iles greenstone belts extending under the north edge of the study area (MacDonald et al. 2005).  Archean intrusive rocks consist predominately of trondhjemite-tonalite-grandiorite and granite with minor felsic to mafic intrusions of the ~2.69 Ga sanukitoid suite (e.g. Lac des Iles) (Heaman et al. this volume).  The northern portion of the study area is underlain by interbanded mafic metavolcanic rocks and clastic to chemical metasedimentary that occur along the southern margin of the subprovince, and have been interpreted by Williams (1987) to represent an imbricated thrust wedge developed by the collision between the Wabigoon and Wawa subprovinces. Position of the Wabigoon-Quetico subprovince boundary is based on the correlation of iron formations underlying the Nipigon Embayment with similar units in the Beardmore–Geraldton greenstone belt indicating that the boundary is located near the southern end of Black Sturgeon Lake (e.g. Sutcliffe 1986). The Quetico Subprovince is composed of <2.68 Ga (Percival and Sullivan 1988) metamorphosed turbiditic wackes interpreted to be largely derived from the neighbouring Wabigoon and Wawa subprovinces (e.g. Williams 1989). Amphibolites are more common towards the south as a result of increasing metamorphic grades, and are intruded by irregular bodies of biotitic leucogranite or biotite gneiss producing complex mixtures classified as migmatite.  A number of bands within the metawackes are intruded by a high density of irregular coarse-grained to pegmatitic bodies and dykes of muscovite granite.

Equivalents to three units located in the northwest portion of the Nipigon Embayment have not been identified within the study area, but may be present beneath the later Mesoproterozoic rocks.  The Badwater Intrusion is a ~1.59 Ga mafic to ultramafic body intruded by slightly younger felsic dykes, and is located northwest of English Bay on Lake Nipigon (Fig.  XX) (Heaman et al. this volume; MacDonald 2004). A series of flat-lying sequence of mafic pillowed flows and hyoclastites overlie the Badwater Intrusion, and are intruded by a >1.159<1.514 Ga diabase sill (Heaman et al. this volume; MacDonald 2004).  The 1.547 Ga English Bay volcanic-intrusive complex (Heaman et al. this volume; Hollings et al. 2004) is located on the northwest show of Lake Nipigon.  These units appear to be localized along a series of north-trending faults can be traced for over 100 km south, in part controlling deposition of the sedimentary and igneous rocks of the study area.

The Group is composed of 5 formations (Rogala 2003), with the Pass Lake, Rossport and Kama Hill, present within the area examined by the AMT survey (e.g. Hart 2005).  The Pass Lake Formation consists of the conglomeratic Loon Lake Member and the commonly sheet-like sandstones of the Fork Bay Member, representing a braided fluvial to shallow lacustrine environment (e.g. Cheadle 1986).  The Rossport Formation consists of cyclic dolomite–red shales of the Channel Island Member, the stromatolitic Middlebrun Bay Member, and red shaly siltstone with mud-chip conglomerates and sand sheets of the Fire Hill Member. Cheadle (1986) interpreted the Channel Island, Middlebrun Bay, and Fire Hill members to have been deposited in a playa lake, strandlines formed during regression of the lake, and a sabkha environment respectively.  Rogala et al. (2005) interpreted the change to a sabkha environment to represent a change in tectonic style from broad subsidence to active half-graben formation, which occurred more than 200 million years before the Midcontinent Rift.  The Kama Hill Formation is composed of shales and siltstones, with sandstones more abundance upward in the sequence. Rogala et al. (2005) interpreted this formation to represent reflooding of the basin, followed by deposition of deltaic distributary mouth bar complexes of the Outan Island Formation and eolian sandstones of the Nipigon Bay Formation both which are not present in the area of the survey.  Bedding in the Sibley Group is generally flat-lying on an outcrop scale, but up to 200 m of topographic relief on the Archean paleosurface was noted by Cheadle (1986).  Stratigraphy is thickest along the hinge line of the half-graben, represented by the Black Sturgeon River, and thins towards the west (e.g. Coates, 1972).  Although generally weakly metamorphosed, up to 10 m wide zones of hornfels metamorphism are variably developed in the Sibley Group in contact with the peridotites, and occasionally the diabase sills.

Four mafic to ultramafic intrusions ranging in age from 1.106–1.124 Ga (Heaman et al. this volume) were emplaced into the Sibley Group and underlying Archean rocks. All four intrusions are composed of a pyroxene peridotite core with an irregular olivine gabbro zone along the margin (e.g., Hart et al., 2002; Hart and Magyarosi, 2004). The Seagull Intrusion has been studied in the greatest detail, and petrography indicates that it consists predominately of cumulate textured dunites, lherzolites, and olivine websterite, with minor olivine- hornblende pyroxenite (Heggie 2005). An olivine gabbro forms an irregular border zone to the ultramafic rocks, and is the unit most commonly exposed in outcrop. The present geometry of the intrusions is sill-like covering areas ranging between 20 and 85 km2 with thicknesses of 130 to 730 m thick. The Seagull Intrusion is the largest of the intrusions with a central saucer-shaped portion that may be a product of post-intrusion faulting or a result of emplacement into a pre-existing structure, but an underlying, or peripheral, feeder structure has not yet been identified for this, or any of the other intrusions. Ultramafic hosted platinum group element (PGE) mineralization is located in the Seagull Intrusion within discrete, laterally continuous zones (e.g. Platinum Group Metals 2005).  A basal zone of mineralization, present near the lower contact of the intrusion, is interpreted to be caused by sulphur saturation of the magma during initial stages of emplacement, and zones higher in the intrusion probably reflect influxes of less evolved magma (Heggie 2005). The Disraeli Intrusion is geochemically and geochronologically similar to the Seagull Intrusion (Richardson et al. this volume; Heaman et al. this volume), and was examined in detail as part of this study. Diabase sills intrude into, and have chill contacts with, units of three of these intrusions.

A number of fine-grained, massive mafic sills, with higher olivine contents than the diabase, intrude the Sibley Group sedimentary rocks. Some of these sills are spatially associated with the ultramafic intrusions, as in the Moraine Lake and Hele Township areas, and probably represent extensions of the intrusions (Hart, 2005). A sill in the Shillabeer Lake area north of the Mawn Lake Road is not directly related to a larger intrusion (e.g., Richardson et al. this volume), but may correlate with a sill of similar composition that intrudes the upper portion of the Disraeli Intrusion suggesting that some of these mafic sills post-date the larger ultramafic intrusions. These sills are often difficult to distinguish from the diabase sills without petrography and lithogeochemistry.

A series of shallow-dipping 1.11 to 1.114 Ga Nipigon diabase sills, ranging in thickness from less than 5 m to greater than 180 m, intrude all other rock units in the Nipigon Embayment (e.g. Hart 2005, MacDonald 2004. The diabase sills have variable internal subdivisions but are most commonly massive, medium to coarse-grained feldspar and pyroxene with trace to 3% olivine and 1 to 2% magnetite.  Although the rocks generally lacking a sub-ophitic or diabasic texture and should properly be classified as gabbronorite to gabbro, the diabase classification has been applied to all rocks associated with the sills to avoid confusion with other intrusions in the area. Multiple interbanded intervals of very coarse-grained ophitic, coarse-grained, and medium grained diabase define a crude layering that suggests a complex cooling history or injection of more than one phase of magma into the sills with the lack of internal chilled contacts suggesting emplacement of the pulse over a short time period (Hart 2005). There are at least two major sills, greater than 100 m in thickness, which were intersected in diamond drill hole ML01-01 located east of Muskrat Lake (e.g. Hart and Tolson 2005). However, there are no obvious textural, mineralogical, or geochemical differences between the sills that could be used as stratigraphic markers on a map or regional scale, and erosion and block faulting hinders the correlation of the sills and thus the determination of the original number of sills.  South of the Nipigon Embayment, the 1.115 Ga Logan diabase sills intrude metasedimentary rocks of the 1.8 Ga Animikie Group (Heaman et al. this volume).  The Logan sills are geochemically distinct from the Nipigon sills (e.g. Hart, 2003), but are probably part of a single Midcontinent Rift related intrusive event, and are considered to be localized units within the all encompassing Logan igneous suite.

Structure

Formation of the Nipigon Embayment, particularly along the western side, appears to have been the result of the interaction between a series of north- and northwest-trending faults.  The east- to northeast-trending fabric of the underlying Archean rocks appears to have had only a minor effect on the form of the western portion of the Embayment.  However, this fabric may have been more important along the eastern side of the Embayment, in combination with a series of north-trending faults subparallel to the Nipigon River (Sutcliffe 1986).

Interaction between a series of north- and northwest-trending faults is interpreted to be have formed a block faulted asymmetric basin or half-graben (e.g., Coates, 1972). The position of these faults is interpreted based on airborne magnetic and topographic data due to lack of exposure.  The subparallel north-trending faults can be traced for over 150 km, from Seagull Lake to north of English Bay (e.g. MacDonald et al. 2004). Vertical displacements on these faults are variable and in some areas, such as along the Black Sturgeon River, appear to be up to 350 m (Hart 2005).  Lateral displacement appears to be limited but there are few distinctive features that can be traced across the Embayment, other than the iron formations at the south end of Black Sturgeon Lake which do not appear to have been offset (Hart and Magyarosi 2004). The northwest-trending faults, which can be traced for over 50 km into the adjacent central Wabigoon Subprovince. There is no apparent lateral displacement along these faults, but also, there is no unique marker horizon that can be used to determine if any offset has occurred. Vertical displacement across these faults appears to be varied and in some locations, such as between Seagull and Muskrat Lakes, may be the locus of down-dropping of the area to the north (e.g. Hart and Tolson 2005; Rogala et al. 2005).  The proposed sinuous trace for the Black Sturgeon Fault may be a result of interaction between the northwest- and north-trending faults.

Rogala et al. (2005) interpreted that most of the fault activity resulting in the formation of the half-graben was the result of broad subsidence which occurred more than 200 million years before the Midcontinent Rift, based on stratigraphic correlations within the Sibley Group.  Movement along the north-trending faults may have occurred even earlier if the model proposed by Hollings et al. (2004) for the formation of the English Bay Complex as a result of thermal upwelling initiating formation of an intracratonic basin is correct. These faults were probably active during the Keweenawan Midcontinent Rift event as there are highly fractured and altered zones cutting the >1.113 Ga Seagull Intrusion (Heaman et al. this volume) and a 1.090 Ga age on uranium mineralization in north-trending structures east of Black Sturgeon Lake (Ruzicka and LeCheminant, 1984).  Some of these faults are probably deep crustal structures that provided magma conduits for the mafic to ultramafic intrusions and the Nipigon diabase sills.  The Seagull and Disraeli intrusions are located near an intersect between north- and northwest-trending structures, and both intrusion are alignment along structures traceable to the area of the English Bay Complex. These faults also helped to define the shape of the diabase sills, as is particularly evident from the outline of the diabase sills intruding the Wabigoon Subprovince in the Kopka Lake and Garden Lake area, west of the Embayment.

Metavolcanic and metasedimentary rocks of the Archean Wabigoon and Quetico subprovince have a schistosity oriented at approximately 050o to 060o, with near vertical dips.  In the southern portion of the study area, the airborne magnetic survey indicates that the migmatites have a regional fabric oriented at about 080o (Ontario Geological Survey 2004). The felsic intrusive rocks generally follow the trend of the schistosity, except for the bodies that appear to have more irregular contact orientations, but may be controlled by the regional fabric on a larger scale.  Although these structures were not important in the Mesoproterozoic history of the western Embayment, they probably partially controlled the morphology of the paleo-erosional surface on which the Sibley Group was deposited.  The Wabigoon–Quetico subprovincial boundary is tentatively located toward the southern end of Black Sturgeon Lake, as had been proposed by Sutcliffe (1982), and is not defined by a major structure as in the Beardmore (e.g. Hart et al. 2002).

Orientation of the Nipigon diabase sills is controlled by a number of factors including a change in orientation of the principal stress direction (Gretener 1969) during sill emplacement.  Although the easiest path for the sills would be along the basal Archean contact or bedding plane of the Sibley Group, the sills also intrude the Archean rocks and often have contacts dipping up to 30o crossing stratigraphy.  These dipping contacts were observed along north- and northwest-trending ridges (Hart and Magyarosi 2004; Hart 2005) with the most prominent example being the north-trending, east dipping Fox Mountain diabase which partially overlies the Disraeli Intrusion. The three dimension form of the sills is difficult to determine in the field, but  saucer-shaped sills ramping upward along pre-existing structures have been proposed for other mafic sill complexes (e.g., Tasmania: Leaman 1975; Karoo: Chevailler and Woodford 1999) and most spectacularly imaged by seismic survey by Thomson and Hutton (2004) and Planke et al. (2005).  However, these sill complexes have generally been emplaced into sedimentary basins and many of the crudely circular sill outlines observed peripheral to the Nipigon Embayment are hosted by vertically dipping metavolcanic and intrusive rocks of the Wabigoon Subprovince.  Structural data combined with the airborne magnetic pattern (Ontario Geological Survey 2004) for a sill with an approximately 5 km wide, crudely circular plane-view outline, located northeast of the east end of the Mawn Lake Road traverse (Fig. 1), suggests an inwardly dipping saucer-shaped sheet  intruding rocks of the Quetico Subprovince (Hart and Tolson 2005). A similar saucer-shape was interpreted for 8 km wide circular sill in the Garden Lake greenstone belt based on modeling of the airborne magnetic data (Jonathon Rudd, OGS, personal communication, 2001), and also for a semi-circular diabase sill in the Beardmore area (Hart et al. 2002).  Therefore, the number of sills present in the Archean basement to the Nipigon Embayment is probably underestimated due to a lack of diamond drill hole information or geophysical methods that would trace these sills.

# Data Collection and Analysis

The locations used to make MT recordings are shown in Figure 1. Measurements were performed in the fall of 2004 at 837 locations making this one of the largest MT surveys in Ontario. At each site recordings were made of the variations in time of two horizontal components of the electric field and all three (two horizontal plus the vertical) components of the magnetic field.  At one point in the survey there were 4 sets of equipment from the GSC, 4 sets borrowed for the Dublin Institute of Advanced Study and 6 systems from the POLARIS Consortium (www.polarisnet.ca).  This extensive pool of equipment enabled up to 3 crews of 3 persons each to deploy each day.  In general, each crew made time series measurements at 8 sites per day.

At the end of each day, equipment at the last site was left in the field in order to make nighttime measurements.   Nighttime measurements were generally of better quality due to the improvements in the effectiveness of the waveguide (Garcia and Jones, 2002).   The time series were transformed to the Fourier domain and the spectral estimates were used to calculate the ratio of the amplitudes and the phase differences (Fig. 2) of the scattered EM fields.   The transformation to the Fourier domain was typically performed using batch scripts that executed overnight using the recordings made that day and the previous night.  The ratios are normalized to an apparent resistivity, meaning that if the crust were a simple half-space of a certain resistivity it would create the ratios and phase differences observed.  The crust is not a half-space of course and so the apparent resistivity only provides a rough guide to the true resistivity below a site. Because two horizontal components of the electric and magnetic field are measured we are able to generate two curves of apparent resistivity and phase data.   The sites recorded earlier in the fall also generally had better quality data due to the general decline in distant thunderstorm activity as the fall progressed.

Figure 2.  Sample data.  Locations of sites shown in Figure 1.

Recordings were made using a sampling rate of 40000 Hz and with magnetic induction coils sensitive to frequencies higher than only about 1 Hz.  The measured frequency spectrum therefore corresponds roughly to the human audio band and hence these measurements are referred to as audiomagnetotellurics (AMT).  The measured bandwidth is important as shorter periods (or higher frequencies) sample more shallow depths than longer periods (or lower frequencies).  At the overnight sites, induction coils sensitive in a band of approximately a few hundred Hz to over 1000 s were also installed in order to facilitate the calculation of broadband data and to ensure crustal investigation.  The overnight sites were removed the subsequent morning and data up to periods of 100 s or longer in some cases were recorded.  All sites were processed using a remote reference site in order to identify coherent signal and reduce the effects of cultural noise on the magnetic channels.  In general, sources of cultural noise were minimal; however, the Trans Canada pipeline crosses the region near Mawn Lake Road.  The pipeline had a strong manifestation on the vertical field data as expected for a long linear conductor.   No relationship between the apparent resistivity and phase data calculated using horizontal components and the location of the pipeline could be discerned and so it is concluded that the data used in this study are unaffected by the pipeline.

Figure 3.  Mawn Lake Road Pseudosections of apparent resistivity and phase.  The upper two pseudosections represent the so-called XY mode based on data with an electric field polarized along the average strike direction in the histogram for each profile.  The lower pseudosections represent the so-called YX mode data based on the electric field perpendicular the strike.

The geological evidence indicates the dominant shallow lithologies in the area, the sedimentary units and sills, are relatively flat lying.  To evaluate whether the electrical structure is also layered or 1-D, simple examination of the apparent resistivity and phases curves can be made.  If the curves are coincident then that is a strong indication that the crust is layered.   Pseudosections of all the data collected along the Mawn Lake Road profile (Fig. 3) for both the XY and YX mode are generally similar at the higher frequencies.  This observation holds for the data collected along all five profiles (not shown).   This similarity is primary evidence that the shallow electrical structure is predominantly 1-D and that 1-D analytical procedures may be used to generate electrical images of the subsurface.  Occam-1D is a robust method of generating 1-D electrical models based on MT data as it produces the smoothest model that fit the data within a prescribed tolerance (Constable et al., 1987).  After data smoothing and masking to eliminate poor data, the 1-D models were calculated, stitched together along an east-west profile and contoured (Fig. 4) to create a cross-section. The stitched and contoured 1-D models are also reproduced in the PowerPoint file accompanying this report however those were made with heavier smoothing during the final contouring.

Figure 4. Mawn Lake Road cross-section based on 1-D models.   The resistivity stratigraphy on the legend is based on the borehole logging, rock petrophysics and the scientific literature. masl: metres above sea level.

Interpretation

The LNRGI also contracted borehole logging at four locations in the Seagull intrusion for rock property determination (McMahon, 2005).  The log for hole WM04-014 (Fig. 5) demonstrates that resistivity is a useful discriminator of the major units in the Nipigon stratigraphic column.  The Archean basement and Proterozoic ultramafic rocks are, for the most part, resistive (>10 k ohmm).  A surprising feature is the low resistivity within some of the mafic units that may be related to enhanced porosity in these units due to faulting and fracturing.  The Nipigon sills have resistivities of a few k ohmm.   The lowest resistivities (<1 k ohmm) are associated with the Sibley Group.    Table 2 is a summary of laboratory petrophysical measurements on 6 hand samples performed by the petrophysical laboratory at the GSC.  The petrophysical data support the findings of the borehole logging, but they also suggest that the debris flow units of the Sibley Group may have a low resistivity (~85 ohmm) due to their friability, however this result needs further verification.

Figure 5. Borehole resistivity log for WM04-014.  The location of the borehole is shown in Figure 1.

The Mawn Lake Road profile (Fig. 4) is plotted with a colour scale chosen to accentuate the contrast of Sibley sandstone (greenish hues) with more resistive feature such as the Nipigon sills and Archean basement (blue hues).  The sediments clearly thin to the west and appear to have formed in a half-graben basin offset by en echelon faults in a manner predicted by Coates (1972).  The sediments unconformably overly Archean rocks of the Quetico Subprovince and some of the irregularities in the basement profile may represent the topographic relief of the Archean paleosurface which Cheadle (1986) estimated to commonly be 10’s of metres.  Overlain on the section are postulated fault and sill locations.  The sill was intersected in a number of widely space boreholes located on the east half of the profile (e.g. ST02-01, DDH-6; Hart and Magyarosi 2004) so that past correlation was tenuous. The sill appears to migrate up-section as it moves to the west and becomes indistinguishable from the Disraeli Intrusion at approximately km 30 on the profile.  This observation is consistent both with seismic evidence for the migration of sills within shallow sedimentary basins (e.g. Thomson and Hutton 2004; Planke et al., 2005) and finite element modeling that predicts sills will migrate upwards as the sill width approaches that of the depth of emplacement (Malthe-Srenssen et al., 2004; there are other papers that have modeled sill emplacement not by methods (e.g. Pollard et al. 1979) other than finite element but I do not have them at home).  Other mafic sill complexes display similar upward migration of individual sills (e.g., Tasmania: Leaman 1975; Karoo: Chevailler and Woodford 1999), interpreted to be due to a number of factors including a change in orientation of the principal stress direction (Gretener 1969) and following pre-existing structures.  The drill logs in the region reveal the presence of two sills in the area and some of the flattening of the peaks in the deeper portions of the image may reflect this second sill.  More work is required to investigate this dichotomy.  The stitched 1-D models for the other profiles are shown in Appendix 1.

# Future Work

As mentioned earlier the shallow region is predominantly 1-D.  This is not true as the EM waves scatter off the Archean basement at longer periods.   The results of a dimensionality analysis using the McNeice and Jones (2001) implementation of the Groom Bailey decomposition of the MT tensor (Groom and Bailey, 1989) is shown as strike directions in Figure 1.  The best-fit 2-D strike directions are only shown at every fifth site to maximize the visibility of the results.  A rose diagram of the strike directions for each line calculated using data from all site is also shown (Fig. 1).  The absence of a dominant strike direction indicates the region is either 1- or 3-D.  This is the case for the northernmost and southernmost lines (top and bottom histograms on Fig. 1).  The data collected along Eaglenest and Mawn Lake Roads demonstrate preferred strike directions (Fig. 1), albeit the directions are different for each road.  Further, the strike direction preferences appear to align themselves with the surface expression of late faults in the area.  The high frequency data along these profiles vary slowly along the profile line (see Fig. 3 for example) and therefore 1-D models will provide reliable images.   That said, 2-D algorithms are being tested for these lines although the current models remain ‘blocky’ due to memory limitations.  Future plans include breaking the lines up into more manageable sections.

Figure 6.  Current 3-D model.  Plan view location of 3-D model is shown in Figure 1.

1- and 2-D studies do not consider all the data collected at each site. Both studies ignore apparent resistivities and phase difference based on parallel field components. Further, only one of the two modes are utilized in 1-D studies.  All the data are modeled using 3-D algorithms, however these methods are relatively new. To more reliably investigate the region surrounding the Disraeli Intrusion (Fig. 1), the area is currently being modeled using two independent 3-D routines.  The first method is based on forward modeling algorithms (Mackie et al., 1994) utilizing stitched 1-D models calculated form averages of the entire dataset at each site as an initial model.   The model shown in Figure 6 is an inversion result based on an algorithm developed by one of the authors (C.F.) and demonstrates the thinning of the sediments to the west and hints at the 3-D geometry of the intrusion/Sibley interface.   The results of this model study will be published at a later time.

# Conclusions

The shallow portion of the Nipigon Embayment has been successfully investigated using audiomagnetotelluric measurements.   The interpretation of these units is constrained by borehole logging and rock petrophysical data.   The geophysical imaging displayed a sedimentary succession that thins to the west and for the first time reveals the location and magnitude of offset by en echelon faults, the most predominant of which is the Black Sturgeon River fault.  Subsurface Nipigon sills, seen only in spare boreholes, are detected and appear to rise in the sedimentary succession.  Such behavior is consistent with field observations, seismic observations of sills emplaced in other shallow basins, and finite element models of shallow sill emplacement.  Refined images of the subsurface are expected using 2- and 3-D algorithms. AMT is a powerful technique for exploration of the Sibley group and direct detection of ultramafic and other intrusion in the area.  Due to its ability as an electromagnetic geophysical technique to image sulphidic material in addition to sedimentary and mafic packages, AMT is an ideal tool for mineral exploration in the region.

ACKNOWLEDGEMENTS

This work has been funded by the Targeted Geoscience Initiative at the Geological Survey of Canada, the Lake Nipigon Regional Geoscience Initiative, and Lithoprobe.  Tom Muir and Desmond Rainsford provided the hand samples used for the petrophysical analysis. Phil Fralick, and Riku Metsaranta provided invaluable in-field guidance and information regarding the Sibley Group to the first author. Competent field help from the Nipigon band was essential to the success of the data acquisition.  The image of the 3-D model was generated using code developed in part at the Geophysical Inversion Facility at the University of British Columbia.

# REFERENCES

Cheadle, B.A. 1986. Alluvial-playa sedimentation in the lower Keweenawan Sibley Group, Thunder Bay District, Ontario; Canadian Journal of Earth Sciences, v.23, p.527-542.

Chevallier, L., and Woodford, A. 1999. Morpho-tectonic and mechanism of emplacement of the dolerite rings and sills of the western Karoo, South Africa; South African Journal of Geology, v.102, p. 43-54.

Constable, S.C., Parker, R.L, Constable, C.G., 1987, Occam’s Inversion: A practical algorithm for generating smooth models from electromagnetic sounding data, Geophysics,  52, 289-300.

Coates, M.C., 1972, Geology of the Black Sturgeon River Area, District of Thunder Bay, Ontario Department of Mines and Northern Affairs, Geoscience Report 98, 41p.

Devaney, J.R. and Williams, H.R. 1989. Evolution of an Archean subprovince boundary: a sedimentological and structural study of part of the Wabigoon–Quetico boundary in northern Ontario; Canadian Journal of Earth Sciences, v.26, p.1013-1026.

Garcia, X. and Jones, A.G., 2002, Atmospheric sources for audio-magnetotelluric (AMT) sounding, Geophysics, 67, 448-458.

Gretener, P.E., 1969. On the mechanics of the intrusion of sills; Canadian Journal of Earth Sciences, v.6, p. 1415-1419.

Groom, R. and Bailey, R.C., 1989, Decomposition of magnetotelluric impedance tensors in the presence of local three-dimensional galvanic distortion. Journal of Geophysical

Research, 94, 1913-1925.

Hart, T.R. 2003. Keweenawan mafic and ultramafic intrusive rocks of the Lake Nipigon and Crystal Lake areas, northwestern Ontario; abstract in Institute on Lake Superior Geology, Proceedings Volume 49, Part 1-Programs and Abstracts, p.21-22.

Hart, T.R. 2005. Precambrian geology of the southern Black Sturgeon River and Seagull Lake area, Nipigon Embayment, northwestern Ontario; Ontario Geological Survey, Open File Report 6165, 63p.

Hart, T.R. and Magyarosi, Z. 2004. Precambrian Geology of the Northern Black Sturgeon River and Disraeli Lake Area, Northwestern Ontario; Ontario Geological Survey, Open File Report 6138, 56p.

Hart, T.R. and Tolson, A. 2005. Southern Black Sturgeon River–Seagull Lake area geological cross-sections, Nipigon Embayment, northwestern Ontario; Ontario Geological Survey, Preliminary Map P.3563, scale 1:50 000.

Hart, T.R., terMeer, M. and Jolette, C. 2002. Precambrian Geology of Kitto, Eva, Summers, Dorothea and Sandra Townships, Beardmore Area, Northwestern Ontario. Ontario Geological Survey, Open File Report 6095, 206p.

Heggie, G.J. 2005. Whole rock geochemistry, mineral chemistry, petrology and Pt, Pd mineralization of the Seagull intrusion, northwestern Ontario; unpublished MSc thesis, Lakehead University, Thunder Bay, Ontario, 156p.

Hollings, P., Fralick, P. and Kissin, S. 2004. Geochemistry and geodynamic implications of the Mesoproterozoic English Bay granite-rhyolite complex, northwestern Ontario; Canadian Journal of Earth Sciences, v.41, p.1329-1338.

Leaman, D.E., 1975. Form, mechanism, and control of dolerite intrusion near Hobart, Tasmania; Journal of the Geological Society of Australia, v.22, p. 175-186.

Lightfoot, P.C., Sutcliffe, R.H. and Doherty, W. 1991. Crustal contamination identified in Keweenawan Osler Group tholeiites, Ontario: a trace element perspective; Journal of Geology, v.99, p.739-760.

MacDonald, C.A.,2004. Precambrian geology of the south Armstrong – Gull Bay area, Nipigon Embayment, northwestern Ontario. Ontario Geological Survey, Open File Report 6136, 42p.

MacDonald, C.A., Tremblay, E. and Easton, R.M. 2005. Precambrian geology of the west-central map area, Nipigon Embayment, northwestern Ontario: Lake Nipigon Region Geoscience Initiative; Ontario Geological Survey, Open File Report 6164, 49p.

Mackie, R.L., Smith, T.J., and Madden, T.R., 1994, Three-dimensional electromagnetic modeling using finite differences equations: The magnetotelluric example, Radio Science, 29, 923-935.

Malthe-Srenssen, A., Planke, S., Svensen, H., and Jamtveit, B., 2004, Formation of saucer-shaped sills, From: Breitkreuz, C. and Petford, N. (eds) 2004. Physical geology of High-Level Magmatic Systems. Geological Society, London, Special Publications, 234, 215-227.

McMahon, S., 2005, Physical Property Borehole Logging, Logistics Report by DGI Geoscience Inc. for the Lake Nipigon Regional Geoscience Initiative, 9p.

McNeice, G., W. and Jones A.G., 2001, Multisite, multifrequency tensor decomposition of magnetotelluric data, Geophysics, 66, 158-173.

Ontario Geological Survey 2004. Ontario airborne geophysical surveys, magnetic and gamma-ray spectrometer data, grid and vector data, ASCII format, Lake Nipigon Embayment Area; Geophysical Data Set 1047a.

Percival, J. and Sullivan, R.W. 1988.  Age constraints on the evolution of the Quetico Belt, Superior Province, Ontario; in Radiogenic and Isotopic Studies: Report 2, Geological Survey of Canada, Paper 88-2, p.97-107.

Planke, S., Rasmussen, T., Rey, S.S., and Myklebust, R., 2005, Seismic characteristics and distribution of volcanic intrusions and hydrothermal vent complexes in the Vring and Mre basins. In: Dor, A.G. and Vining, B.A. (eds) Petroleum Geology: North-West Europe and Global Perspectives-Proceedings of the 6th Petroleum Geology Conference, 833-844.

Platinum Group Metals Limited, 2005. Platinum Group Metals Limited website: News Release: New Platinum Reef Discovery in Canada, dated January 19, 2005; http://www.platinumgroupmetals.net/s/NewsReleases.asp?ReportID=98017&_Type=News-Releases&_Title=New-Platinum-Reef-Discovery-in-Canada

Rogala, B. 2003. The Sibley Group: a lithostratigraphic, geochemical and paleomagnetic study; unpublished MSc thesis, Lakehead University, Thunder Bay, Ontario, 254p.

Rogala, B., Fralick, P.W. and Metsaranta, R. 2005. Stratigraphy and Sedimentology of the Mesoproterozoic Sibley Group and Related Igneous Intrusions, Northwestern Ontario:  Lake Nipigon Region Geoscience Initiative; Ontario Geological Survey, Open File Report 6174, 87p.

Ruzicka, V. and LeCheminant, G.M., 1984. Uranium deposit research, 1983; in Current Research, Part A, Geological Survey of Canada, Paper 84-1a, p. 39-51.

Thomson K. and Hutton, D. 2004. Geometry and growth of sill complexes: insights using 3D seismic from the North Rockall Trough; Bulletin of Volcanology v.66, p.364-375.

Tomlinson, K.Y., Davis, D.W., Stone, D. and Hart, T.R. 2003. U-Pb age and Nd isotopic evidence for Archean terrane development and crustal recycling in the south-central Wabigoon Subprovince, Canada; Contributions to Mineralogy and Petrology, v.144, p.684-702.

Sutcliffe, R.H. 1982. Precambrian geology of the Wabigoon–Quetico Subprovince Boundary, Grand Bay sheet, Thunder Bay District; Ontario Geological Survey, Preliminary Map P.2528, scale 1:50 000.

Sutcliffe,  R.H., 1986,  Proterozoic rift related igneous rocks at Lake Nipigon, Ontario; unpublished Ph.D thesis, University of Western Ontario, London, Ontario, 325p.

Thomson K. and Hutton, D. 2004. Geometry and growth of sill complexes: insights using 3D seismic from the North Rockall Trough; Bulletin of Volcanology v.66, p.364-375.

Vozoff, K., 1972, The magnetotelluric method in the exploration of sedimentary basins, Geophysics, 37, 98-141.

Williams, H.R. 1987.  Ontario Geoscience Research Grant Program, Grant #242: Structural studies in the Wabigoon and Quetico subprovinces; Ontario Geological Survey, Open File Report 5668, 163p.

Williams, H.R. 1989.  Geological studies in the Wabigoon, Quetico, and Abitibi–Wawa -----subprovinces, Superior Province of Ontario, with an emphasis on the structural development of the Beardmore–Geraldton belt; Ontario Geological Survey, Open File Report 5724, 189p.

Table 1:  Results of rock descriptions and electrical resistivity measurements.

| Sample |  | Lithology | ’ or + (103 m)    |  |  | E |
| --- | --- | --- | --- | --- | --- | --- |
| CGD-2a* | 28hi | Sibley Stromatolitic Limestone Breccia | 12.1 | ----- | ----- | 1.96 |
| CGD-2b* |  |  | 9.79 | ----- | ----- | 10.4 |
| CGD-3a | 28dj | Sibley Rossport Siltstone (debris flow) | 1.15 | 2.00 | 1.61 | 2.0 |
| CGD-3b |  |  | 0.085 | ---- | ---- | 6.7 |
| CGD-5a | 28f | Sibley Rossport Sandstone | 0.43 | 0.46 | 0.39 | 6.0 |
| CGD-6 | 28d | Sibley Rossport Siltstone | 0.42 | 0.37 | 0.36 | 3.1 |
| CGD-9* | 31a | Nipigon Sill (fine grained diabase) | 19.8 | ---- | ---- | 0.65 |
| CGD-12* | 6e | Archean Metasediment | 34.1 | ---- | ---- | 2.6 |

’	= Real electrical resistivity measured after 24 hours saturation.

+	= Amplitude of complex electrical resistivity after 24 hours saturation.

	= Measurement made in the direction perpendicular to bedding (foliation).

, 	= Measurement made in directions parallel with bedding (foliation).

*	= Sample was cut into thinner section because of high electrical resistivity values.

E             = Effective porosity.