Processing of Sturgeon Lake Seismic Reflection Data

for

Noranda Mining and Exploration

by

Brian Roberts and Erick Adam

Geological Survey of Canada

Ottawa, Ontario

January, 1998

Introduction

In November 1997 a 7.8 km seismic line was acquired in the vicinity of Sturgeon Lake, Northern Ontario by J.R.S. Exploration Co., contracted by Noranda Mining and Exploration Inc.  Receivers along the entire line were active for all the shots and 3 seconds 393 channel data were recorded at a sampling rate of 1 ms for each of the 223 dynamite shots.  The details of the acquisition are outlined in Appendix A.  Processing of the seismic data was undertaken at the Seismology and Electromagnetism section of the Geological Survey of Canada with the SEGD data tapes being received on November 25, 1997.  A visual inspection of the shot gathers indicated that the dynamite shots provided ample source energy with strong first break energy along the entire spread of receivers and obvious reflection energy is seen at 1.5 and 2.5 seconds.

Prestack Processing

The prestack processing of the shot gathers focusses mainly on the elimination of noise, both source generated and ambient background, the application of static corrections, and the determination of the correct velocity function for stacking the data (including the application of DMO).  The entire processing flow is detailed in Appendix B, however, below we have listed the highlights of the prestack processing with applicable comments.

1) Geometry definition and application to trace headers

Figure 1 shows the seismic line as defined by the receiver locations, and the cdp processing line as defined by the cdp bin centroids.  Although the seismic line was somewhat crooked. a smooth binning line could be fitted through the midpoints (Figure 2) such that most of the cdp bins contain a full range of offsets.  The cdp fold is as high as 224 in the middle of the line, smoothly tapering to a low fold at the ends of the line (Figure 1).  Elevations vary by only 10 m along the length of the line (Figure 2).

2) True amplitude recovery

A correction for amplitude loss due to spherical divergence was applied using a constant velocity of 6000 m/s [1/(time*vel**2)].  As well, a full trace (3 second) automatic gain function was applied to all traces in order to correct for variations in source and receiver coupling.

3) Minimum phase spiking deconvolution

4) Bandpass filter

The low frequency end of the spectrum primarily contains the dispersive ground roll energy and  air blast (see Figures 3-6) along with some noise related to workings at the mine site and small streams, etc.  To remove this energy the low cut of the bandpass filter was set at 32 Hz.  The  shot gathers also contain abundant high frequency energy, to 200 Hz and beyond, but it was decided that virtually all of the reflective energy in the shot gathers is below 150 Hz.  The bandpass filter was thus set to be quite wide at 32-150 Hz and the frequencies will be revisited again during the post-stack processing.

5) Elevation statics; datum 430 m.

6) 2-D spatial filtering

After deconvolution and bandpass filtering, we are still left with a relatively strong, but discrete, refracted shear wave arrival (head wave).  In order to remove as much of this as possible we applied a median filter which is focussed along a specific velocity (+/- 3450 m/s in this case).  Strong energy at that velocity should be much reduced.  This process was maybe about %80 effective, but we expect that DMO will attenuate the remaining coherent shear wave energy.

7) Mute first break energy

Based on the first break picks.  The data is muted from 30 ms below the first break pick to the top (time 0).

8) Refraction statics application

A 1 layer refraction statics model was defined using the uphole times from the shots and the ProMAX refraction statics package.  Three methods were used for the computations and all 3 came up with comparable results.  The depth of overburden model and the source and receiver statics used to correct for the weathering layer are shown in Figure 7.

9) Residual statics

Surface consistent residual statics were calculated using a 600 ms window centred on the strong package of reflectivity at approximately 1 second.

10) Dip moveout correction (DMO)

In crystalline terranes, both sub-horizontal and dipping events are commonly observed and this is true at Sturgeon Lake.  The dipping events are typically related to steeply dipping contacts and other structures, and diffractions from structural discontinuities and high impedance scatterers.  In applying the normal moveout correction there is no velocity function that will simultaneously optimize stacking of both dipping and sub-horizontal events.  The dipping events stack at anomalously high velocities.  To circumvent this problem, DMO is used to create a section with effectively zero-offset, in which dipping events will stack at realistic velocities.  Prior to DMO an average background velocity (6000 m/s) is used to correct the data, then a full velocity analysis is performed following DMO.

11) Velocity Analysis

Table 1 shows the velocity function used for stacking.

11) CDP stack - straight mean stack

Poststack Processing

12) Bandpass filter

Up to this point we have maintained a wide frequency spectrum in the data.  This was to ensure that every chance was given to enhance the high frequency data, especially in the top .5 s.  In analyzing the data poststack it is observed that most of the coherent energy lies in the 30-80 Hz range, with some higher frequency signal (up to 120 Hz) in the upper part of the section.  Because of this, a time variant bandpass filter was applied at this point (0-.3 s, 30-120 Hz; .3-.45 s, 30-100 Hz; .45-3.0 s, 30-80 Hz).

13) FK migration

Stolt F-K migration with a constant velocity of 6000 m/s (average background velocity).

14) Automatic Gain Control (AGC)

Automatic gain calculation using a 1 s window. Note that tests were also done using a pre-stack AGC with a 500 ms window and omitting an AGC gain altogether and these results are included in this report.

CSP Migration

Common scatter point (CSP) prestack migration, also known as equivalent offset migration (EOM) has been applied to the Sturgeon Lake line using an ITA/Insight poststack module developed at the GSC.  Theory and various application of CSP migration are described by Bancroft et al. (1995).  Our current implementation is limited by two factors: 1) speed (the code is not fully optimised) and 2) there is no means of limiting the migration aperture.  The second factor affect data quality as data points having an aperture close to 90o often include remnants of first break energy, creating high amplitudes on the CSP gathers.  These were attenuated by applying spectral equalisation with a 500 ms sliding window.  While this approach has greatly reduced the noise some is still visible in the top 0.2 s and the data is not "true amplitude".

A total of 694 CSP gathers were assembled at the same location as the CDP line used to produce the DMO section between CDP 176 and 869.  A CSP bin width of 20 m was selected based on receiver spacing and experience.  Each CSP gather consisted of 500 traces with equivalent offsets between 0 and 10 000 m (figure 8).  Note that different bin sizes have not been tested.  The migration velocity is 6.0 km/s (to assemble the CSP gathers) and the maximum migration time is 3 s.  The 3 s maximum migration time and the large offsets (10 km!) were selected to avoid excessive smearing at depth.  Each CSP gather took about 8 min of CPU time on a 200 MHz Sparc Ultra.

The input traces for the CSP migration have been pre-processed using the pre-stack processing flow described above up to the NMO correction step.  After the CSP gathers were assembled, normal moveout corrections and stacking was performed using ITA/Insight software.  A single stacking velocity function was used: 0.0 s - 5500 m/s, 0.2 s - 5900 m/s, 0.5 s - 6000 m/s.  An NMO stretch mute factor of 50 % was selected as a compromise between frequency content at shallow depth and imaging of shallow reflections.  Stack sections using 0 and 30 % stretch mutes have been produced and showed strong sub-horizontal events at the near surface which were probably caused by remnants of the first break picks.

The CSP prestack migrated section of the Sturgeon Lake line is showing a better overall continuity of events between 200 and 3000 ms than the FK migrated section. However, the migrated DMO section shows better images in the first 200 ms and we attribute this difference to the wide aperture of the CSP migration.

While these first results are encouraging there is more to do.  Where the CSP migration is most useful is in determining accurately the velocity field.  Large offsets of the CSP gathers and better signal to noise ratio than standard CMP gathers translate into a more accurate velocity analysis and thus, it should be redone.  The noise on the top of the section could be reduced by limiting the migration aperture.  This will be implemented by developing another ITA/Insight module that will scale the existing CSP gathers using a given aperture range.  Splitting the migration aperture module from the CSP calculation will have the advantage of allowing rapid testing of various migration apertures.

Observations

The data acquired at Sturgeon Lake is of a very high quality and the data processing was fairly routine with no major problems encountered.  We do not have the information required to do a meaningful interpretation and that was not one of the objectives of this project, however there are some comments that can be made about some of the major features displayed on the seismic section.

Below 1 s on the DMO section we see a series of easterly dipping interfaces (20 to 30o) and a strong sub-horizontal reflection package at 2.5 s (figure 9).  The dipping events are of varied strength, but continuous from cdp 700 to the east end of the section.  The exception to this is the lowest layer of dipping reflections which is very strong at the west end of the line, but becomes incoherent at roughly the halfway point (around cdp 500).  Above 1 s the picture is more complex with strong but patchy reflectivity in the west end of the line and a more transparent section to the east (at least above .4 s).  There are at least 2 prominent diffractions that can be identified; one with an apex at station 220 and .5 s (A on figure 9), and one at station 345 and .8 s (B on figure 9).  The first diffraction (.5 s) also appears to have a series of weaker diffractions below it, but they are not so continuous.  Also of note in the top part of the section are some steep easterly dipping reflections (about 40o) between stations 240 and 290 and 150 - 400 ms (C on figure 9).  The sub-horizontal reflections which make up the strong reflectivity in the west part of the line appear to truncate rather abruptly as they extend east.  A hypothetical line joining these truncations (a fault?) would extend to surface at about station 870 (see dashed line on figure 9).

The migrated sections (csp and fk) are noisier and contain more artifacts than the DMO section, however, they clarify the picture somewhat in the upper section.  Below 1 s the events are as described above except that the dipping events have moved up dip to the west and appear to flatten towards the east end of the line.  Above 1 s we see that the 2 diffractions mentioned above have collapsed to fairly discrete zones of moderate to high amplitude reflections.  On the fk migrated section the steeply dipping reflections between stations 240 and 290 have steepened up and now we can project them from the surface at approximately station 295 to sub-horizontal reflections at .5-.6 s and stations 210-220.  Again, the sub-horizontal events to the west end of the line appear to truncate against some east-dipping plane.

Conclusions

We have observed strong reflectivity at Sturgeon Lake and some potentially interesting structures, both at depth and in the top .5 seconds.  The final stacks (DMO structure stack and migrated stack) could possibly be improved by more effort in at least 2 areas.

1) The refraction statics solution appears to be at least adequate, but it is possible that a better solution could be arrived at through the use of a more robust refraction statics program (like GLI3D).  It's possible that this would lead to an improved image in the upper section.  We may investigate getting this done.

2) The velocity analysis could be redone using the common scatter point gathers.  This would result in minor tuning of the velocities leading to a more accurate picture of the true velocities, and perhaps a slightly better image.

References

Bancroft, John: Prestack Migration by Equivalent Offset and Common Scatter Point (CSP) Gather (1995).  Available from: http://www.crewes.ucalgary.ca/Papers/index.html

Table 1.

DESC=rms velocities (dmo)

CDP    =   274.0

TIME   =   55.21  SEMB_VEL= 5883.16

TIME   =  319.02  SEMB_VEL= 6032.67

TIME   =  431.49  SEMB_VEL= 6175.68

TIME   =  705.52  SEMB_VEL= 6234.18

TIME   = 1114.52  SEMB_VEL= 6370.69

CDP    =   324.0

TIME   =   53.17  SEMB_VEL= 5463.89

TIME   =  224.95  SEMB_VEL= 5538.64

TIME   =  398.77  SEMB_VEL= 5847.41

TIME   =  601.23  SEMB_VEL= 6042.42

TIME   =  685.07  SEMB_VEL= 6178.93

TIME   =  944.79  SEMB_VEL= 6234.18

TIME   = 1300.61  SEMB_VEL= 6380.44

TIME   = 1811.86  SEMB_VEL= 6500.70

CDP    =   374.0

TIME   =   51.12  SEMB_VEL= 5639.40

TIME   =  198.36  SEMB_VEL= 5814.91

TIME   =  280.16  SEMB_VEL= 5961.17

TIME   =  519.43  SEMB_VEL= 6188.68

TIME   =  881.39  SEMB_VEL= 6321.94

TIME   = 1094.07  SEMB_VEL= 6351.19

TIME   = 1693.25  SEMB_VEL= 6377.19

CDP    =   424.0

TIME   =   49.08  SEMB_VEL= 5645.90

TIME   =  298.57  SEMB_VEL= 5805.16

TIME   =  478.53  SEMB_VEL= 5961.17

TIME   =  813.91  SEMB_VEL= 6126.93

TIME   = 1144.31  SEMB_VEL= 6162.68

CDP    =   474.0

TIME   =   97.56  SEMB_VEL= 5821.41

TIME   =  211.38  SEMB_VEL= 5964.42

TIME   =  447.15  SEMB_VEL= 6029.42

TIME   =  595.53  SEMB_VEL= 6074.92

TIME   = 1008.13  SEMB_VEL= 6091.18

TIME   = 1747.97  SEMB_VEL= 6081.42

TIME   = 2406.50  SEMB_VEL= 6325.19

CDP    =   524.0

TIME   =   56.91  SEMB_VEL= 5642.65

TIME   =  148.37  SEMB_VEL= 5740.16

TIME   =  290.65  SEMB_VEL= 5847.41

TIME   =  349.59  SEMB_VEL= 5974.17

TIME   =  754.07  SEMB_VEL= 6003.42

TIME   = 1042.68  SEMB_VEL= 6068.42

TIME   = 1888.21  SEMB_VEL= 6084.67

TIME   = 2030.49  SEMB_VEL= 6182.18

TIME   = 2426.83  SEMB_VEL= 6360.94

CDP    =   574.0

TIME   =   54.88  SEMB_VEL= 5639.40

TIME   =  290.65  SEMB_VEL= 5840.91

TIME   =  603.66  SEMB_VEL= 6156.18

TIME   = 1018.29  SEMB_VEL= 6318.69

TIME   = 2439.02  SEMB_VEL= 6386.94

CDP    =   624.0

TIME   =   54.88  SEMB_VEL= 5665.40

TIME   =  207.32  SEMB_VEL= 5863.66

TIME   =  420.73  SEMB_VEL= 5892.91

TIME   =  506.10  SEMB_VEL= 6022.92

TIME   =  745.93  SEMB_VEL= 6139.93

TIME   = 1012.20  SEMB_VEL= 6221.18

TIME   = 1613.82  SEMB_VEL= 6247.18

TIME   = 1869.92  SEMB_VEL= 6312.19

TIME   = 2331.30  SEMB_VEL= 6338.19

TIME   = 2739.84  SEMB_VEL= 6403.19

CDP    =   674.0

TIME   =   50.81  SEMB_VEL= 5658.90

TIME   =  182.93  SEMB_VEL= 5749.91

TIME   =  341.46  SEMB_VEL= 5983.92

TIME   =  491.87  SEMB_VEL= 6068.42

TIME   =  790.65  SEMB_VEL= 6227.68

TIME   = 1792.68  SEMB_VEL= 6237.43

TIME   = 2298.78  SEMB_VEL= 6305.69

TIME   = 2607.72  SEMB_VEL= 6422.69

CDP    =   724.0

TIME   =   42.68  SEMB_VEL= 5642.65

TIME   =  241.87  SEMB_VEL= 5866.91

TIME   =  382.11  SEMB_VEL= 6019.67

TIME   =  593.50  SEMB_VEL= 6087.93

TIME   =  747.97  SEMB_VEL= 6175.68

TIME   = 1764.23  SEMB_VEL= 6354.44

TIME   = 2508.13  SEMB_VEL= 6393.44

CDP    =   774.0

TIME   =   44.72  SEMB_VEL= 5665.40

TIME   =  140.24  SEMB_VEL= 5840.91

TIME   =  296.75  SEMB_VEL= 5915.67

TIME   =  394.31  SEMB_VEL= 6003.42

TIME   =  524.39  SEMB_VEL= 6058.67

TIME   =  597.56  SEMB_VEL= 6143.18

TIME   =  743.90  SEMB_VEL= 6234.18

TIME   =  961.38  SEMB_VEL= 6370.69

TIME   = 1721.54  SEMB_VEL= 6497.45

TIME   = 2449.19  SEMB_VEL= 6529.95

CDP    =   824.0

TIME   =   58.94  SEMB_VEL= 5707.65

TIME   =  195.12  SEMB_VEL= 5996.92

TIME   =  282.52  SEMB_VEL= 6175.68

TIME   =  477.64  SEMB_VEL= 6208.18

TIME   =  766.26  SEMB_VEL= 6357.69

TIME   = 1243.90  SEMB_VEL= 6422.69

TIME   = 1709.35  SEMB_VEL= 6481.20

Appendix A

Appendix B.

SEG-D Input

Inline Geom Header Load

Primary header to match database              FFID

True Amplitude Recovery

Basis for spherical spreading               1/(time*vel**2)

Specify TAR velocity function

0-6000/

APPLY function to data or REMOVE effect       Apply

of amplitude corrections?

Maximum application TIME                      3000.

Automatic Gain Control

Application mode                              APPLY

Type of AGC scalar                            MEAN

AGC operator length                           3000.

BASIS for scalar application                  Centered

Robust Scaling?                               No

Spiking/Predictive Decon

TYPE of deconvolution                         Minimum phase spiking

Decon operator length(s)                      120

Operator 'white noise' level(s)               0.1

Time gate reference                           Time 0

Get decon gates from the DATABASE?            Yes

SELECT decon gate parameter file            decon_gate2

Output traces or filters                      Normal decon output

Apply a bandpass filter after decon?          Yes

Bandpass filter frequency values            27-32-150-160

Apply Elevation Statics

Final datum elevation                         430.

Replacement velocity                          5900.

Database math method                          Shot Holes Using Uphole Info

NMO static method                             Elevations

Length of smoother                          51

Processing DATUM                              FINAL DATUM

2-D Spatial Filtering

Type of 2-D filter                            Simple Alpha-trimmed Mean

Rejection percentage for spatial            50.

filter

Number of SAMPLES for 2-D filter              7

Number of TRACES for 2-D filter               7

Application mode for 2-D filter               Subtraction

Steer filters along a velocity dip?           Yes

Linear moveout velocity for dip             3450.

steering

Trace muting

Database/Header Transfer

Direction of transfer                         Load TO trace header FROM database

First header entry                            First break pick time

Trace Header Math

Select mode                                   Fixed equation mode

DEFINE trace header equation

tlive_s=REAL(FB_PICK)+25.0

Trace Header Math

Select mode                                   Fixed equation mode

DEFINE trace header equation

tfull_s=REAL(FB_PICK)+35.0

Trace Header Math

Select mode                                   Fixed equation mode

DEFINE trace header equation

tlive_e = 3000.0

Trace Muting

Re-apply previous mutes                       Re-mute

Trace Muting

Re-apply previous mutes                       Re-ramp

Apply Refraction Statics

Final datum elevation                         430.000000

Replacement velocity                          5900.000000

NMO static method                             Mean Statics

Length of smoother                            51

Processing DATUM                              FINAL DATUM

Apply Residual Statics

Type of residual statics to apply           Max. Power Autostatics

Normal Moveout Correction

Direction for NMO application                 FORWARD

Stretch mute percentage                     50.

Apply any remaining static during           Yes

NMO?

Get velocities from the database?             No

SPECIFY NMO velocity function(s)

1:0-5500,200-5900,500-6000/

Ensemble DMO in T-X Domain

CDP interval                                  10.

Typical CDP spacing in ensembles              10.

Header entry to use for offsets               Signed source-receiver offset

Normal Moveout Correction

Direction for NMO application                 INVERSE

Get velocities from the database?             No

SPECIFY NMO velocity function(s)

1:0-5500,200-5900,500-6000/

Normal Moveout Correction

Direction for NMO application                 FORWARD

Stretch mute percentage                     50.

Apply any remaining static during           Yes

NMO?

Get velocities from the database?             Yes

SELECT Velocity parameter file              rms velocities (dmo)

CDP/Ensemble Stack

Sort order of input ensembles                 CDP

METHOD for trace summing                      Mean

Root power scalar for stack                   0.5

normalization

Apply final datum statics after stack?        No

Bandpass Filter

TYPE of filter                                Time and Space-Variant Filter

Type of filter specification                  Ormsby bandpass

PHASE of filter                               Minimum

Percent additive noise factor               1.

Apply a notch filter?                         No

Space-variant filter parameters

1:0:25-30-120-130,25-30-100-110,25-30-80-85/

Get time gates from the DATABASE?             No

SELECT Primary time gate header word        CDP bin number

SELECT Secondary time gate header           No trace header entry selected

word

SPECIFY filter time gate parameters

1:0:0-300,350-450,500-3000/

Memory Stolt F-K Migration

Maximum frequency  to migrate (in Hz)         90.

RMS velocities for migration

1:0-6000/

Number of traces to smooth velocity           0

field over

Percent velocity scale factor                 100.

Stolt stretch factor                          1.

Re-apply trace mutes?                         No

Re-kill dead traces?                          No