Table 1 - Data Acquisition Parameters

1.1 Original Survey Parameters

Survey area approx. 30 km2

15x25 m subsurface bin size

13 receiver lines

30 m receiver station spacing

300 m receiver line spacing

6 source lines

50 m source point spacing

600 m source line spacing

1050 shot points (0.25 - 0.5 kg in 5-10 m deep holes)

1.2 Intrumentation

2000-channel, 24-bit telemetry system I/O System II

High Cut Filter: 270/188 Hz

Pre-Amp Gain: 36 dB

Geophone Type: Mark L-210 (marsh phone)

Geophone Array: bunched

3-component downhole geophone SIE T42 locked at 1070 m depth

All channels live

helicopter assisted deployment and pick-up of equipment